# KAIROS: Cryptographic Gateway

Technical Specification — Module 1

Module 1: Hardware and Firmware Specification

# 1. Purpose and Scope

1.1 Document Purpose

This document provides the complete technical specification for the PseudoQuantum Entropy system — a hybrid hardware/software subsystem that captures physical entropy from an optical source and delivers cryptographically usable random bytes to downstream consumers.

It is written primarily for AI-assisted development tools (such as Claude Code, Cursor, or other agentic systems). This document describes what the system must do and how the modules interact. It does not prescribe exact code. Implementation details are left to the developer or AI agent, provided the interfaces, behaviors, and contracts specified here are honored.

1.2 Related Documents

| Document | Description |
| --- | --- |
| Architecture Overview | System-wide diagrams and module interactions |
| Gateway Specification | Downstream cryptographic gateway that consumes entropy (separate document) |
| Python Analyzer Specification | NIST SP 800-22 statistical validation pipeline (separate document) |
| Hardware Architecture Report | Historical record of experimental iterations (reference only) |

1.3 Naming Convention

Throughout this document:

| Term | Refers To |
| --- | --- |
| PseudoQuantum Entropy | The complete hardware-backed entropy generation system |
| Entropy Pool | The C++ module that buffers and serves random bytes |
| Firmware | The ESP32 Arduino sketch that samples and whitens entropy |
| Host | The machine running the C++ backend and Python analyzer |
| Frame | A 2-byte serial packet: [0xAA][DATA_BYTE] |
| Sync Marker | The byte 0xAA used to frame serial packets |
| LSB | Least Significant Bit |
| Von Neumann Extractor | A debiasing technique that discards equal pairs |
| Vref | The 1.65 V midpoint reference used by the analog front end |
| ADC1 | The ESP32 analog-to-digital converter peripheral used by both channels |

1.4 Claim Boundaries (Summary)

The system uses optical fluctuations as the intended entropy source and conditions them for digital random-bit extraction. The measured randomness is not claimed to be purely quantum in origin. A full discussion appears in Section 16.

# 2. System Overview

2.1 Three-Layer Architecture

The PseudoQuantum Entropy system is organized into three layers:

┌─────────────────────────────────────────────────────────────────┐

│                      HARDWARE LAYER                             │

│                                                                 │

│   Dual photodiodes → LM358 amplification → ESP32 ADC            │

│   LSB extraction → Von Neumann whitening → Serial output        │

│                                                                 │

│   Components: Photodiodes, LM358, resistors, capacitor, ESP32   │

└─────────────────────────────────────────────────────────────────┘

│

│ USB Serial (115200 baud)

│ Frame format: [0xAA][DATA_BYTE]

▼

┌─────────────────────────────────────────────────────────────────┐

│                  SERIAL INGESTION LAYER                         │

│                                                                 │

│   SerialEntropySource: opens port, parses frames, recovers      │

│   from sync loss, validates bytes, exposes IEntropySource       │

│                                                                 │

│   Fallback: OpenSSLEntropySource (RAND_bytes)                   │

└─────────────────────────────────────────────────────────────────┘

│

│ Validated entropy bytes

▼

┌─────────────────────────────────────────────────────────────────┐

│                    ENTROPY POOL LAYER                           │

│                                                                 │

│   EntropyPool: thread-safe circular buffer, background          │

│   collection thread, blocking reads, refill threshold,          │

│   serving consumers                                            │

│                                                                 │

│   Consumers: Statistical analyzer, downstream crypto modules    │

└─────────────────────────────────────────────────────────────────┘

2.2 Data Flow

The end-to-end path from optical noise to consumer-usable bytes:

- Optical fluctuation — Ambient light and photon shot noise cause small current fluctuations in the reverse-biased photodiodes.

- Analog amplification — The LM358 op-amps convert and amplify these currents into voltage signals centered at Vref ≈ 1.65 V.

- ADC sampling — The ESP32 samples both ADC channels (GPIO34, GPIO35) at high frequency.

- LSB extraction — Firmware extracts the least significant bit of each reading.

- Von Neumann whitening — Firmware compares pairs of LSBs from the two channels and outputs a bit only when they differ.

- Byte assembly — Eight whitened bits are packed into one byte.

- Serial framing — Each byte is prefixed with a sync marker 0xAA and sent over USB serial.

- Frame parsing — Host-side SerialEntropySource locates sync markers and extracts payload bytes, recovering from misalignment.

- Pool ingestion — Validated bytes are added to the EntropyPool circular buffer by a background collection thread.

- Consumer access — Consumers call getEntropy(n) to receive random bytes; if the pool is below threshold, the collection thread refills it.

- Fallback activation — If the hardware source is unavailable or persistently starved, the pool switches to OpenSSLEntropySource.

2.3 Layer Responsibilities

| Layer | Responsibility | Key Interface |
| --- | --- | --- |
| Hardware | Produce analog voltage signals from optical noise | Photodiode → LM358 → ESP32 ADC pins |
| Firmware | Sample ADC, whiten bits, frame bytes | Serial output: [0xAA][DATA_BYTE] |
| Serial Ingestion | Parse frames, validate sync, expose entropy | IEntropySource::getEntropy() |
| Entropy Pool | Buffer, refill, serve, fallback | EntropyPool::getEntropy() |
| Analyzer | Validate statistical quality | Python subprocess, JSON results |

2.4 Technology Stack

| Component | Technology | Version / Detail |
| --- | --- | --- |
| Microcontroller | ESP32 DevKit V1 | Arduino framework |
| Analog front end | LM358L dual op-amp | DIP-8, single supply 3.3 V |
| Photodiodes | Two-terminal transparent | Reverse-biased |
| Firmware language | C++ (Arduino) | PQHardware.ino |
| Serial baud rate | 115200 | 8-N-1 |
| Host backend | C++17/20 | CMake |
| Serial I/O | Boost.Asio | Cross-platform serial |
| Entropy pool | C++ | Circular buffer, mutex, condition variables |
| Fallback entropy | OpenSSL | RAND_bytes() |
| Configuration | JSON | backend_config.json, logging_config.json |
| Logging | C++ | File + console, INFO/WARN/ERROR |
| Statistical validation | Python | NIST SP 800-22 suite |

2.5 Entropy Budget Overview

The expected entropy throughput of the system is bounded by several stages. Each stage reduces the raw data rate.

| Stage | Nominal Rate | Notes |
| --- | --- | --- |
| ADC sampling | High (kHz range per channel) | Depends on firmware loop timing |
| LSB extraction | 1 bit per ADC reading | Only LSB is used |
| Von Neumann whitening | ~50% of input pairs | Discards 00 and 11 pairs |
| Byte assembly | 8 bits per byte | MSB first |
| Serial framing | 2 bytes per entropy byte | 1 sync + 1 payload |
| Serial baud rate | 115200 baud | ~11,520 bytes/sec raw |
| Effective entropy rate | A few hundred to a few thousand bytes/sec | Depends on whitening efficiency |
| Pool refill | Background thread | Targets refill_threshold |

Exact rates are determined at implementation time. The design guarantees that:

- The pool refills faster than typical consumers drain it.

- The collection thread waits rather than spinning when the buffer is full.

- Starvation triggers OpenSSL fallback after a configurable retry limit.

2.6 Design Constraints

The architecture assumes the following constraints:

- The ESP32 is dedicated to entropy generation; no other workload runs on it.

- Serial communication is point-to-point (ESP32 ↔ host); no multiplexing.

- The host has a stable power supply and does not sleep during entropy collection.

- The pool operates in-memory only; no persistence across restarts.

- The analyzer runs on-demand, not continuously.

- No network access is required for entropy generation.

# 3. Hardware Architecture

3.1 Overview

The hardware layer obtains physical entropy from an optical source and converts photodiode fluctuations into two analog voltage signals suitable for digitization by the ESP32. The analog front end uses two independent optical channels, allowing two separately sampled entropy signals to be combined later in software.

The architecture is:

OPTICAL ENTROPY SOURCE

│

┌──────────┴──────────┐

│                     │

Photodiode 1           Photodiode 2

│                     │

390 kΩ                 390 kΩ

input resistor         input resistor

│                     │

▼                     ▼

LM358 Op-Amp A         LM358 Op-Amp B

│                     │

390 kΩ feedback        390 kΩ feedback

│                     │

▼                     ▼

GPIO34                GPIO35

ADC1                  ADC1

│                     │

└──────────┬──────────┘

▼

Digital entropy

The two channels are physically separate until digital processing. Each channel is a transimpedance-style current-to-voltage converter built around one half of an LM358L dual op-amp.

3.2 Component List (Bill of Materials)

For a complete rebuild, the following components are required. Values are frozen and must not be substituted without re-validating the entropy output.

| Component | Quantity | Final Value / Type | Purpose |
| --- | --- | --- | --- |
| ESP32 DevKit V1 | 1 | ESP32 | ADC and system controller |
| LM358L dual op-amp | 1 | DIP-8 | Analog amplification / current-to-voltage conditioning |
| Photodiode | 2 | Two-terminal transparent photodiodes | Optical entropy source |
| Resistor | 4 | 390 kΩ | Two input resistors + two feedback resistors |
| Resistor | 2 | 150 kΩ | Vref voltage divider |
| Ceramic capacitor | 1 | 0.1 µF | Vref stabilization |
| Breadboard | 1 | Standard solderless breadboard | Circuit assembly |
| Jumper wires | Several | Male/male and/or male/female as required | Electrical connections |
| USB cable | 1 | ESP32-compatible | Power/programming |
| Light source | External | Room light / flashlight during testing | Optical excitation |

Components explicitly not used in the final design:

- No inductor

- No output capacitor

- No separate photodiode pull-down resistor

- No external ADC

The ESP32's own ADCs are used for digitization.

3.3 Photodiode Front End

Channel 1 orientation:

+3.3 V

│

│

Cathode

│

[ PHOTODIODE 1 ]

│

Anode

│

390 kΩ

│

▼

Pin 2  (LM358 IN− A)

Channel 2 orientation: Same as Channel 1 but PIN 6

3.4 LM358 Amplification Stage

Each channel uses one half of the LM358L dual op-amp in a transimpedance-style configuration with negative feedback.

LM358L DIP-8 pinout:

LM358L

┌───────────┐

OUT A ──│ 1       8 │── VCC / 3.3 V

IN− A ──│ 2       7 │── OUT B

IN+ A ──│ 3       6 │── IN− B

GND  ───│ 4       5 │── IN+ B

└───────────┘

Pin assignments:

| Op-amp | Pin | Function |
| --- | --- | --- |
| A | 1 | Output A |
| A | 2 | Inverting input A (−) |
| A | 3 | Non-inverting input A (+) |
| B | 5 | Non-inverting input B (+) |
| B | 6 | Inverting input B (−) |
| B | 7 | Output B |
| Power | 8 | VCC / +3.3 V |
| Power | 4 | GND |

The LM358 family is designed for single-supply operation. The low-voltage variant documentation specifies operation beginning around 3 V, making the 3.3 V supply used here appropriate.

Feedback network:

Each op-amp has a 390 kΩ feedback resistor from output to inverting input:

Pin 1 (OUT A) ─── 390 kΩ ─── Pin 2 (IN− A)

Pin 7 (OUT B) ─── 390 kΩ ─── Pin 6 (IN− B)

3.5 Vref Generation

The op-amps are not referenced directly to ground. Instead, a midpoint reference voltage is generated with a resistive divider.

Divider schematic:

+3.3 V

│

150 kΩ

│

├──────────── Vref ≈ 1.65 V

│

150 kΩ

│

GND

Vref distribution:

Vref ───► Pin 3  (Op-A +)

│

└────► Pin 5  (Op-B +)

Vref stabilization capacitor:

A 0.1 µF ceramic capacitor is connected from the Vref node to ground:

Vref

│

├────────► Pin 3

│

├────────► Pin 5

│

0.1 µF

│

GND

Its purpose is to make the reference node less susceptible to rapid voltage fluctuations and supply/ground noise. This capacitor is kept in the final circuit.

3.6 ESP32 Connections

| ESP32 pin | ADC channel | Function |
| --- | --- | --- |
| GPIO34 | ADC1_CH6 | Photodiode channel 1 |
| GPIO35 | ADC1_CH7 | Photodiode channel 2 |
| 3V3 | Supply | Circuit supply |
| GND | Ground | Common circuit ground |

GPIO34 and GPIO35 are input-only pins and correspond to ADC1 channels 6 and 7 on the classic ESP32. Both channels are on ADC1, not ADC2.

3.7 Complete Power Architecture

All components share a common ground.

ESP32

┌────────────┐

│            │

3V3 ──┤            ├── GND

└─────┬──────┘

│

┌────────────┴─────────────┐

│                          │

▼                          ▼

LM358 Pin 8               LM358 Pin 4

3.3 V                    GND

3.8 Pin-to-Pin Connection Table

This is the authoritative reference for breadboard assembly.

| From | To | Purpose |
| --- | --- | --- |
| ESP32 3V3 | LM358 Pin 8 | Op-amp supply |
| ESP32 GND | LM358 Pin 4 | Common ground |
| ESP32 3V3 | Photodiode 1 cathode | Reverse bias |
| ESP32 3V3 | Photodiode 2 cathode | Reverse bias |
| Photodiode 1 anode | 390 kΩ | Channel 1 input path |
| 390 kΩ | LM358 Pin 2 | Channel 1 inverting input |
| LM358 Pin 1 | 390 kΩ | Channel 1 feedback |
| Feedback 390 kΩ | LM358 Pin 2 | Channel 1 feedback |
| LM358 Pin 1 | GPIO34 | Channel 1 ADC |
| Photodiode 2 anode | 390 kΩ | Channel 2 input path |
| 390 kΩ | LM358 Pin 6 | Channel 2 inverting input |
| LM358 Pin 7 | 390 kΩ | Channel 2 feedback |
| Feedback 390 kΩ | LM358 Pin 6 | Channel 2 feedback |
| LM358 Pin 7 | GPIO35 | Channel 2 ADC |
| 3.3 V | 150 kΩ | Vref divider top |
| First 150 kΩ | Vref node | Divider midpoint |
| Vref node | Second 150 kΩ | Divider midpoint to ground |
| Second 150 kΩ | GND | Vref divider return |
| Vref | LM358 Pin 3 | Channel 1 reference |
| Vref | LM358 Pin 5 | Channel 2 reference |
| Vref | 0.1 µF capacitor | Reference stabilization |
| 0.1 µF capacitor | GND | Reference stabilization |

3.9 Design Rationale and Frozen Decisions

The following decisions are frozen. Future developers or AI agents should not change them without explicit re-validation of entropy quality.

Why 390 kΩ input and feedback resistors

Lower resistance produced a small output response (insufficient amplification). Higher resistance produced larger amplification but saturated under bright illumination. 390 kΩ was settled on as the working compromise after experimental iteration.

Why no output capacitor

A 0.1 µF capacitor from op-amp output to ground was experimentally tested. It produced worse behavior — slower output and larger/slower variations instead of the fast fluctuations needed for entropy. It was removed.

Key lesson: Filtering is not automatically beneficial for an entropy source. A filter can suppress unwanted electrical noise but also suppress or distort the fast fluctuations useful to entropy extraction.

Why two photodiodes

Not simply to produce "twice as much light." The two channels provide two separate physical measurements. The software system can combine bits from each channel, and the Von Neumann extractor relies on comparing LSBs from two independently sampled channels. Two channels also allow investigation of whether combining them improves statistical output.

Why ADC1

Both ADC inputs are on ADC1. ADC2 is shared with the Wi-Fi subsystem on the classic ESP32 and cannot be used reliably while Wi-Fi is active. This is a frozen ESP32 constraint.

Why LSB extraction

Only the least significant bit of each ADC reading is used. Higher bits are dominated by the DC operating point, ambient light level, and supply drift. The LSB is the noisiest and most entropic portion of the sample.

Why Von Neumann whitening

Raw LSB samples may be biased or temporally correlated. The Von Neumann extractor removes first-order bias by discarding equal pairs (00, 11) and outputting only when the pair differs. This is a minimal, well-understood debiasing method that does not require a cryptographic hash.

# 4. Firmware Specification

4.1 Overview

The firmware runs on the ESP32 DevKit V1 and is responsible for:

- Sampling both analog entropy channels

- Extracting the least significant bit from each ADC reading

- Applying Von Neumann whitening to remove bias

- Assembling whitened bits into bytes

- Transmitting each byte over serial with a sync marker

- Driving status LEDs to indicate entropy activity

The firmware is the only software running on the ESP32. No other workload, network stack, or peripheral is used. This is a frozen constraint: the ESP32 is dedicated to entropy generation.

Target file: hardware/PQHardware.ino
Language: C++ (Arduino framework for ESP32)

4.2 Firmware Responsibilities and Non-Responsibilities

The firmware shall:

- Initialize the serial port at the specified baud rate

- Configure the two ADC input pins

- Configure the two status LED pins

- Continuously generate entropy bytes

- Frame and transmit each byte over serial

- Update LED status based on entropy activity

- Never block indefinitely

- Never write anything other than framed entropy bytes to the serial stream

The firmware shall not:

- Perform cryptographic hashing or whitening beyond Von Neumann

- Store entropy in persistent memory

- Communicate over Wi-Fi, Bluetooth, or any network interface

- Respond to commands from the host

- Validate its own output statistically

- Retry on hardware failure

- Enter any sleep or low-power mode

4.3 Pin Assignments (Firmware View)

| Function | ESP32 GPIO | Direction | Notes |
| --- | --- | --- | --- |
| Entropy channel 1 | GPIO34 | Analog input | ADC1_CH6, input-only pin |
| Entropy channel 2 | GPIO35 | Analog input | ADC1_CH7, input-only pin |
| Green LED | GPIO4 | Digital output | Active high |
| Red LED | GPIO5 | Digital output | Active high |
| Serial TX | Default UART TX | Output | USB serial to host |
| Serial RX | Default UART RX | Input | Not used by firmware |
| Power | 3V3 / GND | — | Supplied via USB |

GPIO34 and GPIO35 are input-only. The firmware must not attempt to drive them.

4.4 ADC Sampling Strategy

Channel selection

Both channels are sampled, but not simultaneously. The ESP32 has a single ADC multiplexer per ADC unit, so channel 1 and channel 2 are sampled sequentially.

Sampling order

For each whitening attempt, the firmware performs:

- Read ADC on channel 1 (GPIO34)

- Wait a short inter-sample delay

- Read ADC on channel 2 (GPIO35)

The two readings form a pair used by the Von Neumann extractor.

Resolution

The ESP32 ADC is used in its default configuration. The firmware does not require a specific resolution to be set; the LSB is extracted from whatever the ADC returns. If a resolution is configured, 12-bit is the natural default for the ESP32.

Inter-sample delay

A short delay (on the order of hundreds of microseconds) is inserted between the two readings. This reduces temporal correlation between the pair, which is important for the Von Neumann extractor to see independent samples.

Sampling rate

The effective sampling rate is determined by:

- ADC conversion time

- Inter-sample delay

- Von Neumann rejection overhead

- Byte assembly overhead

The firmware does not need to target a specific rate. It runs as fast as the loop permits while maintaining the inter-sample delay.

4.5 LSB Extraction

For each ADC reading, the firmware extracts only the least significant bit:

bit = ADC_reading & 1

Behavioral contract:

- The extracted bit must be a single bit (0 or 1).

- No averaging, smoothing, or filtering is applied to the ADC reading before LSB extraction.

- No thresholding or comparison is performed.

4.6 Von Neumann Whitening

The firmware uses a Von Neumann extractor to remove first-order bias from the LSB stream.

Algorithm

For each attempt:

- Read LSB from channel 1 → b1

- Wait inter-sample delay

- Read LSB from channel 2 → b2

- Evaluate the pair (b1, b2):

  - If (0, 1) → output bit 0

  - If (1, 0) → output bit 1

  - If (0, 0) or (1, 1) → discard and retry

4.7 Byte Assembly

Eight whitened bits are packed into one byte.

Bit order

MSB first. The first whitened bit becomes bit 7 of the byte; the eighth becomes bit 0.

Byte value range

Any value from 0x00 to 0xFF is possible. In particular, the byte value 0xAA is possible and is not escaped or remapped. The sync marker and entropy payload share the same byte space.

This is a known property of the frame format (see Section 5) and is handled by the host-side parser through sliding-window sync recovery.

4.8 Serial Output Format

Frame structure

Each entropy byte is transmitted as a 2-byte frame:

| Byte | Value | Description |
| --- | --- | --- |
| 1 | 0xAA | Sync marker |
| 2 | 0x00–0xFF | Entropy payload |

Write order

The sync marker is always written first, immediately followed by the payload byte. No delay is required between the two writes.

No additional framing

The firmware does not add:

- Length headers

- Checksums

- CRCs

- Sequence numbers

- Timestamps

- Line terminators

Serial configuration

| Parameter | Value |
| --- | --- |
| Baud rate | 115200 |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |
| Flow control | None |

Stream behavior

The firmware transmits frames continuously. There is no handshake, no flow control, and no acknowledgement. If the host is not reading, the ESP32 UART buffer will eventually fill and writes will block or drop depending on the UART driver configuration.

The firmware does not detect host absence.

4.9 LED Status Indicators

Two LEDs provide visual feedback that entropy is being generated.

Pin mapping

| LED | GPIO | Meaning |
| --- | --- | --- |
| Green | 4 | Most recent whitened bit was 1 |
| Red | 5 | Most recent whitened bit was 0 |

Update policy

LEDs are updated at a throttled rate (approximately every 100 ms) rather than on every bit. This avoids visible flicker and reduces firmware overhead.

Independence from serial stream

The LED update performs its own bit extraction, independent of the byte that was just transmitted. This means: LED activity has no effect on the bytes sent over serial.

Behavior when no entropy is available

If the Von Neumann extractor is blocked (waiting for a valid pair), LED update is deferred until the next bit is produced. The firmware does not display an error state on the LEDs.

4.10 Main Loop Behavior

Loop structure

The main loop performs the following actions in order:

- Generate one entropy byte via getByte()

- Write sync marker 0xAA to serial

- Write entropy byte to serial

- Check if the LED update interval has elapsed

- If yes, generate one additional whitened bit and update LEDs

- Repeat

No watchdog interaction

The firmware does not feed a watchdog, and no watchdog is expected to be enabled. If a watchdog is enabled by the platform, the loop's regular progress should keep it fed.

4.11 Initialization Sequence

At startup, the firmware shall perform the following in order:

- Initialize the serial port at 115200 baud, 8-N-1.

- Configure green LED pin (GPIO4) as digital output.

- Configure red LED pin (GPIO5) as digital output.

- Optionally set ADC attenuation (implementation choice; must be consistent between channels).

- Optionally perform a short warm-up delay to let the analog front end settle.

- Enter the main loop.

The firmware does not wait for a host connection. It begins transmitting entropy frames as soon as the main loop begins

4.12 Timing and Throughput Considerations

Sources of delay

| Source | Approximate contribution |
| --- | --- |
| ADC conversion | Microseconds per reading |
| Inter-sample delay | Hundreds of microseconds |
| Von Neumann rejection | ~50% overhead on average |
| Byte assembly | Negligible |
| Serial write | Bounded by UART buffer and baud rate |

Expected effective rate

At 115200 baud, the UART can transmit approximately 11,520 bytes per second. Each entropy byte consumes 2 bytes on the wire (sync + payload), giving a maximum payload rate of ~5,760 bytes per second if the UART is the bottleneck.

Backpressure

If the host stops reading, the ESP32 UART transmit buffer will fill. Behavior depends on the Arduino-ESP32 serial implementation.

4.13 Interface Contracts

Hardware → Firmware

The firmware assumes the following from the hardware layer:

| Signal | Expectation |
| --- | --- |
| GPIO34 | Analog voltage centered near Vref ≈ 1.65 V, fluctuating with optical input |
| GPIO35 | Analog voltage centered near Vref ≈ 1.65 V, fluctuating with optical input |
| 3.3 V rail | Stable within ESP32 operating tolerance |
| GND | Common ground with LM358 and photodiodes |

The firmware does not detect hardware failures (e.g., disconnected photodiode, saturated op-amp). Such conditions produce degraded entropy that is detected downstream by the analyzer

Firmware → Host

The firmware guarantees the following to the host:

| Guarantee | Meaning |
| --- | --- |
| Frame integrity | Each entropy byte is preceded by exactly one 0xAA marker |
| Continuous stream | Frames are emitted as fast as the firmware can produce them |
| No gaps | No pause markers, no idle bytes, no protocol negotiation |
| No reordering | Bytes are emitted in the order produced |
| No duplication | Each byte is transmitted exactly once |

The firmware does not guarantee:

- A specific byte rate

- Statistical quality of the bytes (that is the analyzer's responsibility)

- Recovery from UART errors (the host parser handles this)

- Acknowledgement of receipt

4.14 Failure Modes and Firmware Behavior

| Failure | Firmware behavior |
| --- | --- |
| Photodiode disconnected | ADC reading floats; LSB may still vary due to noise; entropy continues but quality degrades |
| LM358 output saturated | ADC reading clips at rail; LSB may become constant; entropy quality degrades |
| Serial host not reading | UART buffer fills; firmware blocks or drops depending on driver |
| ADC not responding | Firmware behavior undefined; typically reads return 0 or last value |
| Power brownout | ESP32 resets; firmware restarts from initialization |

The firmware does not attempt to detect or recover from any of these conditions. Detection and fallback are the responsibility of the host-side entropy pool (Section 6) and analyzer (Section 8)

4.15 Firmware Design Constraints

The following constraints are frozen:

- No cryptographic hashing on the ESP32.

- No network or wireless functionality.

- No persistent storage.

- No command interface.

- No dynamic configuration.

- No firmware-side statistical tests.

- No escaping of the 0xAA byte in the payload.

- No changes to the frame format without updating the host parser.

Any change to these constraints requires re-validation of the serial protocol (Section 5) and the entropy pool's frame parsing behavior (Section 6)

# 5. Serial Communication Protocol

5.1 Overview

The protocol is deliberately minimal:

- Fixed 2-byte frames

- No headers, no length fields, no checksums

- No handshake, no acknowledgement, no flow control

- Unidirectional: ESP32 transmits, host receives

This minimalism is a frozen decision. The protocol is designed so that the ESP32 firmware stays simple and the host parser handles all error recovery.

5.2 Physical Layer

| Parameter | Value |
| --- | --- |
| Transport | USB Serial (UART over USB) |
| Baud rate | 115200 |
| Data bits | 8 |
| Parity | None |
| Stop bits | 1 |
| Flow control | None |
| Direction | ESP32 → host only |
| Host → ESP32 | Not used by the protocol |

The host may open or close the serial port at any time. The ESP32 does not detect host presence and continues transmitting regardless.

5.3 Frame Format

Every entropy byte is transmitted as a fixed 2-byte frame:

┌──────────────┬──────────────┐

│   SYNC       │   PAYLOAD    │

│   0xAA       │   0x00–0xFF  │

│   1 byte     │   1 byte     │

└──────────────┴──────────────┘

| Field | Size | Value | Description |
| --- | --- | --- | --- |
| Sync | 1 byte | 0xAA | Frame delimiter, always present |
| Payload | 1 byte | 0x00–0xFF | One whitened entropy byte |

Frame properties:

- Frames are uniform. Every frame is exactly 2 bytes.

- There is no length header because frame length is fixed.

- There is no frame counter, timestamp, or sequence number.

- The payload byte may itself equal 0xAA. It is not escaped or remapped.

5.4 Sync Marker

The sync marker is the single byte 0xAA.

Purpose:

- Marks the start of a new frame

- Allows the host parser to align to frame boundaries after any desynchronization

5.5 Payload

The payload is one whitened entropy byte produced by the firmware (Section 4.7).

| Property | Value |
| --- | --- |
| Size | 1 byte |
| Value range | 0x00–0xFF |
| Distribution | Intended to be uniform |
| Bias | Removed by Von Neumann extractor in firmware |
| Statistical validation | Host-side analyzer (Section 8) |

5.6 Frame Validation

Validation rule: A frame is considered valid if and only if:

- The byte at the expected frame-start position equals 0xAA.

- The next byte is treated as the payload without further validation.

There is no checksum, CRC, parity byte, or length check.

Why no length field: Frames are fixed length. A length field would be redundant.

5.7 Frame Parsing (Host Side)

The host-side parser is implemented by SerialEntropySource (Section 6.4). Its behavior is specified here as part of the protocol contract.

Normal operation. When synchronized, the parser reads two bytes at a time:

read byte → must be 0xAA → discard (sync)

read byte → payload → emit to entropy pool

repeat

5.8 Sync Loss Detection

Definition. Sync loss occurs when the byte read at the expected frame-start position is not 0xAA.

Causes:

| Cause | Description |
| --- | --- |
| Port opened mid-stream | The parser starts reading at an arbitrary byte offset |
| USB buffer corruption | Rare, but possible under host stress |
| ESP32 reset mid-frame | A frame may be truncated |
| Payload misalignment | A 0xAA payload byte read at the wrong offset can shift phase |

5.9 Sync Recovery

Recovery algorithm. When sync is lost, the parser shifts to a byte-by-byte sliding scan:

- Read one byte.

- If the byte is not 0xAA, discard it and read another.

- When a 0xAA byte is encountered, treat the next byte as the payload.

- Resume normal 2-byte frame cadence.

5.10 False Sync Risk

The problem. The payload byte may equal 0xAA. If the parser is one byte out of phase, a payload 0xAA can be mistaken for a sync marker. A false sync produces a 1-byte misalignment window. The parser reads a payload byte as sync and the next sync byte as payload. This creates one corrupted entropy byte in the output.

Self-correction. The misalignment is self-correcting. After the false sync, the parser reads the real sync marker (0xAA) of the following frame as payload, then reads the next actual sync marker (0xAA) as the next expected sync. Phase is restored within one frame.

5.11 Timeout Behavior

Read timeout. The host-side parser uses a bounded poll timeout (typically 100–1000 ms, configurable). If no bytes arrive within the timeout:

- The read call returns 0 bytes.

- The parser does not emit a payload byte.

- The collector logs a starvation event.

Starvation handling. On timeout:

- The collector yields or sleeps briefly to prevent CPU spinning.

- The source is marked as temporarily unavailable or degraded.

- If timeouts persist consecutively beyond a configurable retry limit, the backend raises an entropy starvation warning.

- The entropy pool falls back to OpenSSLEntropySource (Section 6.5).

5.12 Error Handling Summary

| Error | Detection | Response |
| --- | --- | --- |
| Byte at frame start ≠ 0xAA | Parser check | Enter sync recovery |
| No bytes within timeout | Read returns 0 | Log starvation, mark degraded |
| Persistent timeouts | Consecutive timeout counter | Trigger OpenSSL fallback |
| False sync (0xAA in payload) | Inherent to parser | Self-corrects within one frame |
| Port open failure | OS error on open | Source unavailable; OpenSSL fallback |
| Port closed mid-stream | Read error | Source unavailable; OpenSSL fallback |

5.13 Interface Contract (Firmware → Host)

What firmware guarantees to the host:

| Guarantee | Meaning |
| --- | --- |
| Every frame begins with 0xAA | Sync marker always precedes payload |
| Each frame is exactly 2 bytes | No variable-length frames |
| No extra bytes between frames | Stream is a continuous sequence of 2-byte frames |
| No escaping | Payload may equal 0xAA |
| No handshake required | Host can connect at any time |
| No acknowledgement expected | Host does not reply |
| Continuous transmission | No idle periods unless UART blocks |

What the host does not assume about firmware:

| Non-assumption | Reason |
| --- | --- |
| Fixed byte rate | Depends on ADC timing and rejection rate |
| Statistical quality per byte | Validated by analyzer, not firmware |
| Recovery after UART error | Handled by host parser |
| Detection of host presence | Firmware does not check |
| Ordered delivery across port opens | Each port open may start mid-frame |

5.14 Interface Contract (Host → Firmware)

There is no host-to-firmware channel. The firmware does not read from serial. No configuration commands, no queries, no resets are supported over this interface.

The host may close and reopen the serial port freely. Each open begins mid-stream, and the parser's sync recovery (Section 5.9) handles alignment.

5.15 Constants and Parameters

| Constant | Value | Description |
| --- | --- | --- |
| SYNC_MARKER | 0xAA | Frame delimiter |
| FRAME_SIZE | 2 bytes | Fixed frame length |
| BAUD_RATE | 115200 | Serial baud rate |
| DATA_BITS | 8 | Serial data bits |
| PARITY | None | Serial parity |
| STOP_BITS | 1 | Serial stop bits |
| READ_TIMEOUT_MS | 100–1000 | Host-side read timeout, configurable |
| SYNC_TIMEOUT_MS | Implementation-defined | Max time to wait for a sync marker during recovery |
| STARVATION_RETRY_LIMIT | Implementation-defined | Consecutive timeouts before fallback |

These constants are shared between the firmware (for SYNC_MARKER) and the host parser (for all others). They must remain consistent across both sides.

# 6. Entropy Pool Architecture

6.1 Overview

The entropy pool is a C++ module that receives bytes from an IEntropySource, stores them in a thread-safe circular buffer, and serves them to consumers. It runs a background collection thread that refills the buffer as needed.

The pool is the single point of contact between entropy sources (hardware or software) and all downstream consumers. Consumers never talk to the serial port or OpenSSL directly.

6.2 Module Responsibilities

| Module | Responsibility |
| --- | --- |
| IEntropySource | Abstract interface implemented by all entropy providers |
| SerialEntropySource | Reads ESP32 serial stream, parses frames, recovers sync |
| OpenSSLEntropySource | Provides software fallback via RAND_bytes() |
| EntropyCollector | Selects active source, validates availability, feeds pool |
| EntropyPool | Buffers entropy, refills in background, serves consumers |

6.3 IEntropySource Interface

IEntropySource is the common contract that all entropy providers must implement. It allows the pool to swap between hardware and software sources without changing any consumer code.

Interface methods:

| Method | Returns | Purpose |
| --- | --- | --- |
| initialize() | bool | Set up the source (open port, initialize subsystem) |
| shutdown() | void | Release resources |
| isAvailable() | bool | Check whether source is ready to produce entropy |
| getEntropy(numBytes) | std::vector<uint8_t> | Retrieve N entropy bytes |
| getByte() | uint8_t | Retrieve one entropy byte |
| getSourceName() | std::string | Human-readable name |
| getSourceType() | std::string | Type identifier (e.g., "hardware", "openssl") |
| getTotalBytesGenerated() | size_t | Cumulative bytes produced since initialization |
| resetByteCounter() | void | Reset counter to zero |
| supportsReseeding() | bool | Whether source can be reseeded |
| reseed() | void | Reseed source (if supported) |

Behavioral contract:

- getEntropy(n) returns exactly n bytes or throws EntropyException.

- getByte() returns exactly one byte or throws EntropyException.

- isAvailable() returns false if the source cannot currently produce entropy.

- initialize() must be idempotent-safe: calling it twice should not corrupt state.

- shutdown() must be safe to call even if initialize() was never called.

- reseed() throws EntropyException if supportsReseeding() returns false.

Exception type. EntropyException is a subclass of std::runtime_error used for all entropy-related errors. Its message is prefixed with "Entropy Error: ".

6.4 SerialEntropySource

SerialEntropySource reads the ESP32 serial stream (Section 5) and implements IEntropySource.

Responsibilities:

- Open and configure the serial port

- Send periodic sync requests (not required; firmware streams continuously)

- Parse frames, discarding non-0xAA bytes

- Handle sync loss and recovery

- Detect hardware availability

- Count bytes generated

Construction parameters:

| Parameter | Type | Description |
| --- | --- | --- |
| port | std::string | Serial port path (e.g., COM3, /dev/ttyUSB0) |
| baudRate | unsigned int | Baud rate (115200) |
| timeoutMs | unsigned int | Read timeout in milliseconds |

Serial port configuration:

| Setting | Value |
| --- | --- |
| Baud rate | 115200 |
| Character size | 8 |
| Stop bits | 1 |
| Parity | None |
| Flow control | None |

Availability detection. isAvailable() returns true if and only if:

- The serial port is open

- initialize() has been called successfully

- The most recent connection test passed

If the port cannot be opened, initialize() returns false, but the object remains constructible so that the collector can fall back to another source.

Frame parsing behavior. Specified in Section 5.7–5.10. The parser:

- Reads one byte at a time.

- If the byte is not 0xAA, discards it.

- If the byte is 0xAA, reads the next byte and returns it as payload.

Connection test. On initialize(), the source attempts to synchronize with the stream by scanning for a sync marker within a timeout window (SYNC_TIMEOUT_MS). If a marker is found, the source is marked available. If not, the source is marked unavailable but initialized.

Source name. getSourceName() returns "ESP32 Hardware TRNG (Dual Photodiode)".

Source type. getSourceType() returns "hardware".

Reseeding. Hardware sources do not support reseeding. supportsReseeding() returns false; reseed() throws EntropyException.

Thread safety. getByte() and getEntropy() take an internal mutex to serialize access to the serial port. This prevents concurrent reads from corrupting the stream.

Platform dependency. The implementation depends on Boost.Asio for cross-platform serial I/O. If Boost.Asio is not available at compile time, the source compiles but reports as unavailable; the collector falls back to OpenSSL.

6.5 OpenSSLEntropySource

OpenSSLEntropySource is the software fallback. It uses OpenSSL's RAND_bytes() to fill buffers.

Responsibilities:

- Provide entropy when hardware is unavailable

- Blend with hardware source when both are active (optional)

- Never throw under normal conditions (OpenSSL is expected to always work)

Source name. getSourceName() returns "OpenSSL RAND_bytes".

Source type. getSourceType() returns "openssl".

Availability. isAvailable() returns true as long as OpenSSL is initialized. If OpenSSL is somehow unavailable, it returns false.

Reseeding. supportsReseeding() returns false. OpenSSL's internal state is managed by the library itself.

When it is used. The EntropyCollector decides when to use OpenSSL based on:

- Hardware source initialize() returning false

- Hardware source isAvailable() returning false at runtime

- Persistent starvation from the hardware source

6.6 EntropyCollector

EntropyCollector is the coordinator that selects the active source, detects availability changes, and feeds the pool.

Responsibilities:

- Instantiate the configured entropy source(s)

- Call initialize() on each

- Select the active source according to configuration (auto, hardware, openssl)

- Monitor source availability

- Trigger fallback when needed

- Push bytes into the EntropyPool

Source selection modes:

| Mode | Behavior |
| --- | --- |
| auto | Try hardware first; fall back to OpenSSL if unavailable |
| hardware | Use only the hardware source; fail if unavailable |
| openssl | Use only OpenSSL (for testing or fallback scenarios) |

Fallback trigger conditions. The collector switches from hardware to OpenSSL when any of the following are true:

- Hardware initialize() fails

- Hardware isAvailable() returns false after a read attempt

- The number of consecutive read timeouts exceeds STARVATION_RETRY_LIMIT

- Hardware throws EntropyException during getEntropy()

Recovery from fallback. If the hardware source becomes available again after a fallback (e.g., USB reconnected), the collector may switch back to hardware. Whether this recovery is automatic or requires a restart is a configuration choice.

Validation. The collector does not perform statistical validation of bytes. Validation is the responsibility of the Python analyzer (Section 8). The collector only validates structural correctness (frame format, sync markers) via SerialEntropySource.

6.7 EntropyPool

EntropyPool is the thread-safe circular buffer that stores entropy and serves it to consumers.

Construction parameters:

| Parameter | Type | Description |
| --- | --- | --- |
| source | std::shared_ptr<IEntropySource> | The entropy provider |
| bufferSize | size_t | Circular buffer size in bytes |
| refillThreshold | size_t | Refill trigger level in bytes |

If source is null, the constructor throws EntropyException.

If refillThreshold exceeds bufferSize, it is clamped to bufferSize.

Public methods:

| Method | Returns | Purpose |
| --- | --- | --- |
| start() | bool | Start background collection thread |
| stop() | void | Stop thread and release resources |
| getEntropy(numBytes) | std::vector<uint8_t> | Blocking read of N bytes |
| getByte() | uint8_t | Blocking read of one byte |
| tryGetEntropy(numBytes, buffer) | size_t | Non-blocking read |
| available() | size_t | Current byte count in buffer |
| isRunning() | bool | Whether collection thread is active |
| isEmpty() | bool | Whether buffer is empty |
| isFull() | bool | Whether buffer is full |
| getBufferSize() | size_t | Configured buffer size |
| setRefillThreshold(threshold) | void | Change refill threshold |
| getRefillThreshold() | size_t | Current refill threshold |
| refill() | bool | Manually trigger a collection cycle |
| setSource(source) | void | Replace the entropy source |
| getSourceName() | std::string | Name of active source |
| getTotalBytesGenerated() | size_t | Cumulative bytes collected |

Internal state:

| Field | Type | Purpose |
| --- | --- | --- |
| buffer | std::vector<uint8_t> | Circular storage |
| head | size_t | Read position |
| tail | size_t | Write position |
| count | size_t | Bytes currently in buffer |
| bufferSize | size_t | Capacity |
| refillThreshold | size_t | Refill trigger |
| mutex | std::mutex | Protects all state |
| cv | std::condition_variable | Signals readers when bytes are available |
| refillCV | std::condition_variable | Signals collector when refill is needed |
| collectorThread | std::thread | Background collection |
| running | bool | Thread loop flag |
| stopped | bool | Pool has been stopped |
| totalBytesGenerated | size_t | Cumulative byte count |
| entropySource | std::shared_ptr<IEntropySource> | Active source |

Collection thread behavior. The background thread performs the following loop:

while running:

if needsRefill():

collectEntropy()    # pull from source, add to buffer

else:

sleep(COLLECTION_INTERVAL)

wait for refill signal or 500 ms timeout

Constants:

| Constant | Value | Meaning |
| --- | --- | --- |
| COLLECTION_INTERVAL | Implementation-defined | Sleep duration when buffer is healthy |
| COLLECTION_BATCH_SIZE | Implementation-defined | Number of bytes requested per source read |

Refill logic. needsRefill() returns count < refillThreshold && running.

The collector thread refills when:

- The buffer falls below the threshold

- The refillCV is signaled by a reader that consumed bytes

- The periodic wait times out and the buffer is still below threshold

Reader behavior. getEntropy(numBytes) performs the following:

- Take mutex.

- While count < numBytes and pool is running and not stopped, wait on cv.

- If pool is stopped, throw EntropyException.

- If insufficient bytes after wake, throw EntropyException.

- Read numBytes bytes from the buffer.

- If refill is now needed, notify refillCV.

- Return the bytes.

Non-blocking read. tryGetEntropy(numBytes, outBuffer) performs the following:

- Take mutex.

- If pool is stopped or not running, return 0.

- Read min(numBytes, count) bytes.

- Append to outBuffer.

- If refill is now needed, notify refillCV.

- Return number of bytes read.

Buffer management. The circular buffer is accessed via head (read position) and tail (write position). Both wrap modulo bufferSize.

Writing (addBytes):

- Assumes mutex is held.

- Writes min(data.size(), bufferSize - count) bytes.

- Advances tail modulo bufferSize.

- Increments count.

- Notifies all readers via cv.

Reading (readBytes):

- Assumes mutex is held.

- Reads min(numBytes, count) bytes.

- Advances head modulo bufferSize.

- Decrements count.

- Notifies collector via refillCV if refill is now needed.

Source replacement. setSource(source) allows the active source to be replaced at runtime. The mutex is held during replacement. A change is logged.

6.8 Threading Model

Threads:

| Thread | Role |
| --- | --- |
| Main thread | Owns the pool object; calls start(), stop(), consumer reads |
| Collection thread | Background refill loop |
| External consumer threads | May call getEntropy() concurrently |

Synchronization primitives:

| Primitive | Purpose |
| --- | --- |
| mutex | Protects all pool state |
| cv | Readers wait here for bytes |
| refillCV | Collector waits here for refill requests |

Locking discipline:

- All public methods take mutex before touching state.

- Internal helpers (addBytes, readBytes, needsRefill) assume mutex is already held.

- cv.wait() is used with std::unique_lock<std::mutex>.

- refillCV.wait_for() is used with std::unique_lock<std::mutex>.

- The collection thread does not hold the mutex during sleep_for or during source I/O.

6.9 Fallback Decision Tree

START POOL

Is source configured as "auto"? ─── No ───► Use configured source

Yes: Try hardware initialize()

├── Success ──► Is hardware available()? ── Yes ──► Use hardware

No:  Fall back to OpenSSL

└── Failure ──► Use OpenSSL

AT RUNTIME

Hardware read timeouts exceed limit?

├── Yes ──► Switch to OpenSSL, log warning

No: Error Handling

6.10 Error Handling

| Error | Detection | Response |
| --- | --- | --- |
| Null source in constructor | Parameter check | Throw EntropyException |
| Source unavailable at start | isAvailable() | start() returns false |
| Collection failure | Exception during getEntropy() | Log, sleep 100 ms, retry |
| Pool stopped | running flag | Throw EntropyException on read |
| Insufficient bytes | count < numBytes | Block until enough or stopped |
| Source replacement with null | Parameter check | Throw EntropyException |

6.11 Configuration Parameters

The following parameters are drawn from backend_config.json:

| Config key | Default | Purpose |
| --- | --- | --- |
| entropy.source | "auto" | Source selection mode |
| entropy.pool_buffer_size | 4096 | Circular buffer size |
| entropy.pool_refill_threshold | 2048 | Refill trigger threshold |
| serial.port | "COM3" | Serial port path |
| serial.baud_rate | 115200 | Baud rate |
| serial.timeout_ms | 5000 | Read timeout |

6.12 Interface Contract (Entropy Pool → Consumers)

Consumers of the entropy pool are guaranteed:

| Guarantee | Meaning |
| --- | --- |
| Thread-safe reads | Multiple consumers may call getEntropy() concurrently |
| Blocking semantics | getEntropy(n) returns exactly n bytes or throws |
| Byte integrity | Bytes returned are exactly those collected from the source |
| No duplication | No byte is returned to two different consumers |
| No reordering | Bytes are returned in the order they entered the pool |
| Exception on stop | After stop(), reads throw EntropyException |
| Eventual fallback | If hardware fails, software fallback is used transparently |

Consumers are not guaranteed:

| Non-guarantee | Reason |
| --- | --- |
| Fixed byte rate | Depends on source production |
| Statistical quality per byte | Validated by analyzer, not by pool |
| Persistence across restarts | Pool is in-memory only |
| Fixed buffer occupancy | Refills are demand-driven |

# 7. Configuration

7.1 Overview

Configuration for the entropy subsystem is provided through JSON files that are shared between modules. These files define the structures that the backend, the serial ingestion layer, and the analyzer all agree on. They are the single source of truth for runtime parameters.

The configuration layer is deliberately simple:

- Two JSON files: backend_config.json and logging_config.json

- Loaded at startup by the backend

- No database, no environment-specific schemas, no remote configuration

- No hot-reload required (though optional)

The configuration files are considered part of the module interface contract. Any AI-assisted implementation must produce and consume the same structure

7.2 Backend Configuration File

File: config/backend_config.json

This file defines the runtime parameters for the backend, serial ingestion, entropy pool, and analyzer integration.

Full structure:

json

{

"serial": {

"port": "COM3",

"baud_rate": 115200,

"timeout_ms": 5000

},

"entropy": {

"source": "auto",

"pool_buffer_size": 4096,

"pool_refill_threshold": 2048

},

"http": {

"port": 8080

},

"analysis": {

"test_mode": "quick",

"test_size": 1024

},

"server": {

"frontend_path": "./frontend",

"max_upload_size": 10485760

},

"crypto": {

"key_size": 256,

"salt_size": 32,

"iterations": 100000

}

}

7.3 Backend Configuration Fields

serial section:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| port | string | "COM3" | Serial port path. Platform-dependent: COM3, COM4, … on Windows; /dev/ttyUSB0, /dev/ttyACM0, … on Linux/macOS |
| baud_rate | integer | 115200 | Serial baud rate. Must match firmware |
| timeout_ms | integer | 5000 | Read timeout in milliseconds. Used by SerialEntropySource |

entropy section:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| source | string | "auto" | Source selection: "auto", "hardware", or "openssl" |
| pool_buffer_size | integer | 4096 | Circular buffer size in bytes |
| pool_refill_threshold | integer | 2048 | Refill trigger level in bytes. Must be ≤ pool_buffer_size |

analysis section:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| test_mode | string | "quick" | Analyzer mode: "quick" or "full" |
| test_size | integer | 1024 | Number of entropy bytes to collect for analysis |

7.4 Logging Configuration File

File: config/logging_config.json

This file defines logging behavior for all backend modules, including entropy source, collector, and pool.

Full structure:

json

{

"log": {

"file": "logs/backend.log",

"level": "INFO",

"console_output": true

}

}

Fields:

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| file | string | "logs/backend.log" | Path to log file, relative to backend working directory |
| level | string | "INFO" | Log level: "DEBUG", "INFO", "WARN", "ERROR" |
| console_output | boolean | true | Whether to also log to console |

7.5 Entropy-Relevant Log Events

The entropy subsystem produces the following log events. These are not configured but are guaranteed by the implementation.

| Event | Level | Source | Description |
| --- | --- | --- | --- |
| Source initialized | INFO | SerialEntropySource | Serial port opened successfully |
| Source unavailable | WARN | SerialEntropySource | Serial port could not be opened or connection test failed |
| Source degraded | WARN | SerialEntropySource | Read timeout or sync failure |
| Sync loss detected | DEBUG | SerialEntropySource | Frame misalignment detected |
| Sync recovered | DEBUG | SerialEntropySource | Sync marker found after loss |
| Fallback activated | WARN | EntropyCollector | Switching to OpenSSL due to hardware failure |
| Fallback recovery | INFO | EntropyCollector | Switching back to hardware |
| Pool started | INFO | EntropyPool | Collection thread started |
| Pool stopped | INFO | EntropyPool | Collection thread stopped |
| Refill triggered | DEBUG | EntropyPool | Buffer below threshold |
| Pool starvation | WARN | EntropyPool | Consumers blocked waiting for entropy |
| Collection error | ERROR | EntropyPool | Source threw exception during collection |

Log format is JSON lines to the log file and human-readable to console. Exact format is implementation-defined but must include timestamp, level, module, and message.

7.6 Configuration Loading Behavior

Load sequence:

- Backend starts.

- Reads config/backend_config.json.

- Reads config/logging_config.json.

- Validates both files.

- Initializes logger.

- Initializes entropy subsystem using serial and entropy sections.

- Initializes analyzer integration using analysis section.

Failure behavior:

| Failure | Response |
| --- | --- |
| backend_config.json missing | Backend fails to start with error |
| backend_config.json invalid JSON | Backend fails to start with parse error |
| Required field missing | Backend fails to start with field name |
| Field value out of range | Backend clamps to valid range and logs WARN, or fails, depending on field |
| logging_config.json missing | Default logging used (INFO to console, no file) |
| logging_config.json invalid | Default logging used, startup continues |

Default fallback: If a value is missing but the section exists, the default value from the tables in Section 7.3 is used. If the entire section is missing, all defaults for that section are used.

7.7 Configuration Validation Rules

| Rule | Applies To | Behavior on Violation |
| --- | --- | --- |
| baud_rate must match firmware | serial.baud_rate | Error at startup; only 115200 is valid |
| timeout_ms must be > 0 | serial.timeout_ms | Clamp to 1 ms, log WARN |
| source must be one of auto, hardware, openssl | entropy.source | Error at startup |
| pool_buffer_size must be ≥ 1 | entropy.pool_buffer_size | Error at startup |
| pool_refill_threshold must be ≤ pool_buffer_size | entropy.pool_refill_threshold | Clamp to pool_buffer_size, log WARN |
| test_mode must be quick or full | analysis.test_mode | Error at startup |
| test_size must be > 0 | analysis.test_size | Clamp to 1, log WARN |
| level must be valid log level | log.level | Default to INFO, log WARN |

7.8 Configuration File Locations

| Environment | Path |
| --- | --- |
| Development | ./config/backend_config.json and ./config/logging_config.json |
| Production | /etc/pseudoquantum/backend_config.json and /etc/pseudoquantum/logging_config.json |

The backend resolves paths relative to its working directory. The server.frontend_path field defines the frontend asset root, which is out of scope here

7.9 Interface Contract (Configuration → Modules)

Modules that read configuration are guaranteed:

| Guarantee | Meaning |
| --- | --- |
| File structure is stable | JSON layout will not change without version bump |
| Field names are frozen | Renaming requires updating all consumers |
| Defaults are defined | Missing values fall back to defaults |
| Validation is deterministic | Same invalid value always produces same behavior |
| Load happens before use | No module reads config after startup |

Modules are expected to:

| Expectation | Meaning |
| --- | --- |
| Read only their relevant section | Entropy modules do not read http, server, or crypto |
| Log when defaults are used | Missing values should be logged at WARN level |
| Fail fast on invalid required values | Do not silently continue with bad config |
| Not write to config files at runtime | Configuration is read-only after startup |

# 8. Statistical Validation

8.1 Overview

The statistical validation subsystem is a Python-based analyzer that consumes raw entropy bytes and evaluates them against standard statistical test suites. It is the primary mechanism for verifying that the hardware and firmware produce output with the expected randomness properties.

The analyzer is not part of the real-time entropy path. It runs on demand, typically:

- During initial integration testing

- After hardware changes

- Before deployment

- Periodically during operation (optional)

The analyzer never feeds back into the entropy pool. Its results are informational and diagnostic.

8.2 Analyzer Architecture

┌──────────────────────────────────────────┐

│         C++ Backend (host)               │

│                                          │

│  EntropyPool.getEntropy(test_size)       │

│         │                                │

│         ▼                                │

│  Spawn Python subprocess                 │

│         │                                │

│         │ stdin: raw entropy bytes       │

│         │ stdout: JSON results           │

└─────────┼────────────────────────────────┘

│

▼

┌──────────────────────────────────────────┐

│         Python Analyzer                  │

│                                          │

│  entropy_analyzer.py                     │

│    ├── reads bytes from stdin            │

│    ├── selects test mode                 │

│    ├── invokes nist_tests.py functions   │

│    ├── assembles JSON result             │

│    └── writes JSON to stdout             │

│                                          │

│  nist_tests.py                           │

│    ├── statistical test implementations  │

│    └── returns per-test pass/fail        │

└──────────────────────────────────────────┘

The analyzer is invoked as a subprocess. Communication is via stdin/stdout using raw bytes in and JSON out. No persistent service is required.

8.3 Analyzer Components

| Component | File | Responsibility |
| --- | --- | --- |
| Analyzer entry point | analyzer/entropy_analyzer.py | Accept input, select mode, invoke tests, emit JSON |
| Test implementations | analyzer/nist_tests.py | Individual statistical tests |
| Dependencies | analyzer/requirements.txt | Python package dependencies |
| Result schema | analyzer/test_results_schema.json | JSON schema for output |

8.4 Test Modes

The analyzer supports two modes, configured via analysis.test_mode.

Quick mode. A small set of fast-running tests intended for frequent use during development and integration.

| Property | Value |
| --- | --- |
| Purpose | Fast sanity check |
| Runtime | Seconds for typical input sizes |
| Tests | A small subset of statistical checks |
| Input size | Typically 1024 bytes or more |
| When used | Development, integration, routine health checks |

Full mode. A comprehensive evaluation using the complete NIST SP 800-22 test suite.

| Property | Value |
| --- | --- |
| Purpose | Thorough statistical validation |
| Runtime | Longer; scales with input size |
| Tests | Full NIST SP 800-22 suite |
| Input size | Larger samples required (typically 100 KB or more) |
| When used | Pre-deployment validation, periodic deep analysis |

Exact test lists are not frozen in this document. The analyzer implementation may select specific tests within each mode as long as:

- Quick mode uses a representative subset

- Full mode uses the standard NIST SP 800-22 suite

- Each test reports a pass/fail result with a p-value

8.5 Analyzer Invocation

Trigger. The backend invokes the analyzer when:

- A REST endpoint for analysis is called (out of scope here)

- A manual test is requested during development

- A scheduled periodic check runs (optional, not required)

Invocation contract:

python analyzer/entropy_analyzer.py --mode <quick|full> --size <N>

| Argument | Value | Description |
| --- | --- | --- |
| --mode | quick or full | Test mode |
| --size | integer | Number of entropy bytes to analyze |

Input. Raw entropy bytes are written to the analyzer's stdin.

- Format: raw binary

- Length: exactly --size bytes

- No framing, no headers, no encoding

Output. The analyzer writes JSON results to stdout.

- Format: JSON

- Schema: see Section 8.7

- Exit code: 0 on success, non-zero on error

8.6 Input Data Format

| Property | Value |
| --- | --- |
| Source | EntropyPool.getEntropy(test_size) |
| Format | Raw binary bytes |
| Length | As specified by --size |
| Encoding | None |
| Metadata | None (raw bytes only) |

The analyzer is agnostic to the source of the bytes. It does not know or care whether they came from hardware or OpenSSL fallback.

8.7 Output Data Format

The analyzer produces a JSON object on stdout. The exact fields may vary by mode, but the following structure is required.

Minimal output schema:

json

{

"status": "success",

"mode": "quick",

"size": 1024,

"tests": [

{

"name": "test_name",

"passed": true,

"p_value": 0.532,

"details": {}

}

],

"summary": {

"total_tests": 4,

"passed": 4,

"failed": 0,

"overall": "pass"

}

}

Field descriptions:

| Field | Type | Description |
| --- | --- | --- |
| status | string | "success" or "error" |
| mode | string | "quick" or "full" |
| size | integer | Number of bytes analyzed |
| tests | array | One entry per statistical test |
| summary | object | Aggregate results |

Per-test fields:

| Field | Type | Description |
| --- | --- | --- |
| name | string | Test name (implementation-defined) |
| passed | boolean | Whether the test passed |
| p_value | number | p-value from the test, if applicable |
| details | object | Test-specific additional data |

Summary fields:

| Field | Type | Description |
| --- | --- | --- |
| total_tests | integer | Total number of tests run |
| passed | integer | Number of tests that passed |
| failed | integer | Number of tests that failed |
| overall | string | "pass" or "fail" |

Error output:

json

{

"status": "error",

"message": "Human-readable error message",

"mode": "quick",

"size": 0

}

8.8 Acceptance Criteria

The analyzer is expected to pass all tests in the selected mode for a hardware entropy source under normal operating conditions.

| Criterion | Threshold |
| --- | --- |
| Individual test p-value | ≥ 0.01 (standard NIST criterion) |
| Proportion of passing tests | ≥ 96% (typical NIST recommendation) |
| Overall result | All tests must pass for "acceptable" status |

Exact thresholds may be adjusted during implementation. The standard NIST criterion is p-value ≥ 0.01 for individual tests.

Interpretation:

- A single failing test is not necessarily a problem. Random sequences occasionally fail tests at the expected rate.

- Multiple failures indicate a genuine problem with the entropy source.

- Persistent failures under the same conditions indicate bias, correlation, or insufficient entropy.

8.9 Failure Handling

| Scenario | Analyzer behavior | Backend behavior |
| --- | --- | --- |
| Python not installed | Exit non-zero with error | Log error, do not affect entropy generation |
| Analyzer script missing | Exit non-zero with error | Log error |
| Insufficient input bytes | Exit non-zero with error | Log error |
| Test execution fails | Mark test as failed with detail | Report in JSON result |
| Overall result is "fail" | Return "overall": "fail" | Log warning, do not stop entropy generation |

The analyzer never halts the entropy subsystem. A failing statistical test is an informational diagnostic, not a control signal. The decision to switch to OpenSSL fallback is made by EntropyCollector based on serial availability, not on analyzer results.

8.10 Analyzer Configuration

Analyzer parameters are drawn from backend_config.json:

| Config key | Default | Purpose |
| --- | --- | --- |
| analysis.test_mode | "quick" | Analyzer mode |
| analysis.test_size | 1024 | Number of bytes to analyze |

Additional configuration is provided on the command line:

| Argument | Default | Purpose |
| --- | --- | --- |
| --mode | from config | Overrides config mode |
| --size | from config | Overrides config size |

8.11 Analyzer Dependencies

The analyzer requires:

| Dependency | Purpose |
| --- | --- |
| Python 3.8+ | Runtime |
| NumPy | Array operations for test implementations |
| SciPy | Statistical functions (erfc, chi-squared, etc.) |
| Standard library | JSON, file I/O, subprocess |

Exact dependency versions are specified in analyzer/requirements.txt. The analyzer must run on the same host as the backend without requiring network access.

8.12 Interface Contract (Backend → Analyzer)

The backend is guaranteed by the analyzer:

| Guarantee | Meaning |
| --- | --- |
| Deterministic invocation | Same input + same mode produces same result |
| Structured output | JSON output is always emitted on stdout |
| Exit code signaling | 0 on success, non-zero on failure |
| No side effects | Analyzer does not modify entropy pool or files |
| No network access | All processing is local |
| No persistent state | Each invocation is independent |

The analyzer is guaranteed by the backend:

| Guarantee | Meaning |
| --- | --- |
| Input is raw bytes | No headers, no framing, no encoding |
| Input length matches --size | Backend provides exactly the requested length |
| Input is from a single source | Not a mix of hardware and fallback in one run |
| No concurrent invocations | Backend does not run two analyzer instances simultaneously |

# 9. Error Handling and Resilience

9.1 Overview

The core philosophy is:

- Fail fast. Detect errors early and clearly.

- Fail graceful. Degrade functionality; do not crash the host.

- Recover automatically. Retry when possible; fall back when not.

- Log everything. Every error event is recorded for debugging.

- Never block indefinitely. Timeouts bound every read operation.

The entropy subsystem is designed so that no single hardware or software failure stops the host process. If the hardware source is lost, the pool falls back to OpenSSL. If the analyzer fails, entropy generation continues. If configuration is invalid, the backend refuses to start rather than running with undefined behavior.

9.2 Error Categories

Errors are grouped by the layer where they originate.

| Category | Origin | Examples |
| --- | --- | --- |
| Hardware errors | Analog front end | Photodiode disconnected, op-amp saturation, supply droop |
| Firmware errors | ESP32 firmware | UART buffer full, ADC read failure, reset mid-frame |
| Serial errors | Physical link and host parser | Port open failure, sync loss, read timeout |
| Entropy source errors | IEntropySource implementations | Initialization failure, availability loss, exception during read |
| Entropy pool errors | EntropyPool | Null source, pool stopped, insufficient bytes |
| Collector errors | EntropyCollector | Source selection failure, fallback trigger |
| Configuration errors | Startup config loading | Missing file, invalid JSON, out-of-range value |
| Analyzer errors | Python subprocess | Python not installed, test failure, invalid input |
| Lifecycle errors | Startup and shutdown | Thread spawn failure, join failure, signal handling |

Each category is handled at the layer where it originates, with defined escalation behavior.

9.3 Hardware Errors

Hardware errors are the most difficult to detect because the firmware does not monitor the analog front end. These errors manifest as degraded entropy quality rather than explicit failure signals.

| Scenario | Detection | Immediate Effect | Downstream Response |
| --- | --- | --- | --- |
| Photodiode disconnected | Not detected by firmware | ADC reading floats; LSB may still vary | Analyzer detects bias or reduced entropy |
| Photodiode reverse-biased incorrectly | Not detected | ADC reading constant near rail | Analyzer detects failing tests |
| LM358 output saturated | Not detected | ADC reading clips; LSB may become constant | Analyzer detects bias |
| Vref drift | Not detected | ADC midpoint shifts | Analyzer detects deviation from uniform distribution |
| Power supply droop | ESP32 may brown out | Firmware resets; stream stops | Serial timeout; fallback triggered |
| Ambient light saturation | Not detected | ADC reading near top rail | Analyzer detects bias |
| Complete darkness | Not detected | ADC reading near bottom rail | Analyzer detects bias |

Hardware error policy:

- The firmware does not attempt to detect or correct hardware errors.

- Detection is the responsibility of the Python analyzer (Section 8).

- Persistent analyzer failures are an operational signal to inspect the hardware.

- No automatic fallback is triggered by analyzer results. Fallback is triggered only by serial availability loss.

9.4 Firmware Errors

| Scenario | Detection | Firmware Behavior | Host Effect |
| --- | --- | --- | --- |
| UART transmit buffer full | Arduino-ESP32 driver | Write blocks or drops depending on driver | Stream stalls or loses frames; parser handles via sync recovery |
| ADC read returns invalid value | Not detected | Firmware uses whatever the ADC returns | LSB may still be random or constant |
| Von Neumann extractor stalls | Firmware blocks | Loop waits indefinitely for a valid pair | Serial stream stalls; host times out |
| ESP32 reset mid-frame | Host parser | Truncated frame | Parser enters sync recovery |
| ESP32 watchdog reset | ESP32 hardware | Firmware restarts | Stream pauses briefly, then resumes |
| Power brownout | ESP32 hardware | Firmware restarts | Same as watchdog reset |

Firmware error policy:

- The firmware has no error reporting channel.

- Any firmware failure appears to the host as a stream anomaly.

- The host parser and collector handle all firmware-originated failures.

9.5 Serial Errors

Serial errors are handled by SerialEntropySource (Section 6.4) and the EntropyCollector.

| Error | Detection | Response | Escalation |
| --- | --- | --- | --- |
| Port cannot be opened | OS error on open() | initialize() returns false | Collector falls back to OpenSSL |
| Port closed mid-stream | Read error | isAvailable() returns false | Collector falls back to OpenSSL |
| Byte at frame start ≠ 0xAA | Parser check | Enter sync recovery | None |
| No bytes within timeout | Read returns 0 | Log starvation; mark degraded | Fallback after retry limit |
| Persistent timeouts | Consecutive timeout counter | Raise starvation warning | Trigger OpenSSL fallback |
| False sync (0xAA in payload) | Inherent to parser | Self-correct within one frame | None |
| USB disconnect | OS-level error | Read fails | Fallback to OpenSSL |

Serial error policy:

- Transient sync loss is handled silently by the parser.

- Transient timeouts are logged and retried.

- Persistent timeouts trigger fallback.

- Port open failure triggers immediate fallback.

9.6 Entropy Source Errors

Each IEntropySource implementation reports errors through isAvailable() and by throwing EntropyException.

SerialEntropySource errors:

| Error | Method | Behavior |
| --- | --- | --- |
| Port open failure | initialize() | Returns false, logs WARN |
| Connection test failure | initialize() | Returns false, logs WARN |
| Read timeout | getByte(), getEntropy() | Throws EntropyException |
| Serial I/O error | getByte(), getEntropy() | Throws EntropyException |
| Sync not found | getByte() | Throws EntropyException after SYNC_TIMEOUT_MS |
| Reseed attempt | reseed() | Throws EntropyException |

OpenSSLEntropySource errors:

| Error | Method | Behavior |
| --- | --- | --- |
| OpenSSL not initialized | initialize() | Returns false |
| RAND_bytes() failure | getEntropy() | Throws EntropyException |
| Reseed attempt | reseed() | Throws EntropyException |

Entropy source policy:

- Sources never return fewer bytes than requested.

- Sources never return zero bytes for a non-zero request.

- Sources throw EntropyException rather than returning invalid data.

- Sources are responsible for their own internal state consistency.

9.7 Entropy Pool Errors

| Error | Detection | Response |
| --- | --- | --- |
| Null source in constructor | Parameter check | Throw EntropyException |
| start() called when source unavailable | isAvailable() | Returns false, logs WARN |
| Pool stopped while readers blocked | running flag | Wake readers, throw EntropyException on read |
| Insufficient bytes after wake | count < numBytes | Throw EntropyException |
| Collection failure | Exception in collection thread | Log ERROR, sleep 100 ms, retry |
| setSource() with null | Parameter check | Throw EntropyException |
| stop() called twice | running flag check | No-op, no error |
| start() called twice | running flag check | Returns true, no-op |

Entropy pool policy:

- Blocking reads never block indefinitely; they wake on stop.

- All state changes are protected by mutex.

- Collection failures do not stop the collection thread; they are retried.

- The pool never returns partial data for a getEntropy(n) request.

9.8 Entropy Collector Errors

| Error | Detection | Response |
| --- | --- | --- |
| Hardware initialization failure | initialize() returns false | Switch to OpenSSL, log WARN |
| Hardware availability loss at runtime | isAvailable() returns false | Switch to OpenSSL, log WARN |
| Consecutive read timeouts exceed limit | Timeout counter | Switch to OpenSSL, log WARN |
| EntropyException during read | Exception caught | Switch to OpenSSL, log WARN |
| OpenSSL also unavailable | isAvailable() returns false | Log ERROR; pool start returns false |
| Source switches back to hardware | Availability restored | Log INFO, resume hardware |

Collector policy:

- Fallback is automatic and transparent to consumers.

- Recovery from fallback is optional and configurable.

- Every fallback event is logged at WARN level.

- Every recovery event is logged at INFO level.

9.9 Fallback Behavior

The fallback from hardware to OpenSSL is the primary resilience mechanism. Its complete decision logic is:

STARTUP:

source = config.entropy.source

if source == "openssl":

use OpenSSL

done

if source == "hardware":

try hardware initialize()

if success and available:

use hardware

else:

log ERROR

pool start fails

done

if source == "auto":

try hardware initialize()

if success and available:

use hardware

else:

log WARN

use OpenSSL

done

RUNTIME:

while using hardware:

if read timeout count > STARVATION_RETRY_LIMIT:

log WARN

switch to OpenSSL

if isAvailable() returns false:

log WARN

switch to OpenSSL

if EntropyException thrown:

log WARN

switch to OpenSSL

9.10 Graceful Shutdown

| Parameter | Default | Meaning |
| --- | --- | --- |
| STARVATION_RETRY_LIMIT | Implementation-defined | Consecutive timeouts before fallback |
| COLLECTION_INTERVAL | Implementation-defined | Sleep duration when buffer healthy |
| COLLECTION_BATCH_SIZE | Implementation-defined | Bytes per collection attempt |
| READ_TIMEOUT_MS | 100–1000 | Serial read timeout |
| SYNC_TIMEOUT_MS | Implementation-defined | Time to wait for sync marker |

Fallback guarantee: Consumers never see the fallback. A getEntropy(n) call returns valid bytes regardless of which source is active.

9.10 Graceful Shutdown

The entropy subsystem supports clean shutdown in response to process signals.

Shutdown sequence:

- Process receives SIGTERM or SIGINT.

- Backend signals EntropyPool::stop().

- Pool sets running = false, stopped = true.

- Pool notifies cv and refillCV.

- Blocked readers wake and throw EntropyException.

- Collection thread checks running, exits loop.

- Pool joins collection thread.

- SerialEntropySource::shutdown() closes serial port.

- OpenSSLEntropySource::shutdown() releases OpenSSL state.

- Backend exits cleanly.

Shutdown timing: stop() returns only after the collection thread has exited. If the collection thread is blocked in a source read, it will time out within READ_TIMEOUT_MS and then exit

9.11 Retry Strategies

| Operation | Retry Strategy |
| --- | --- |
| Entropy pool call | Blocking call already waits for entropy; no additional retry |
| Serial read timeout | Retry on next collection cycle; increment timeout counter |
| Collection failure | Sleep 100 ms, then retry |
| Analyzer invocation | No retry; report failure to caller |
| Source initialization | No retry; fall back to next source |
| File I/O (config) | No retry at startup; fail fast |
| stop() join | No retry; join is guaranteed to complete |

9.12 Error Codes and Exceptions

The subsystem uses EntropyException as its primary error type. It is a subclass of std::runtime_error, and its message is prefixed with "Entropy Error: ".

Exception sources:

| Source | Example Message |
| --- | --- |
| EntropyPool constructor | "Entropy Error: EntropyPool: Invalid entropy source (nullptr)" |
| EntropyPool::getEntropy() | "Entropy Error: EntropyPool is stopped" |
| EntropyPool::getEntropy() | "Entropy Error: EntropyPool: Insufficient entropy available" |
| EntropyPool::setSource() | "Entropy Error: EntropyPool: Invalid entropy source (nullptr)" |
| SerialEntropySource::getByte() | "Entropy Error: Serial entropy source not available" |
| SerialEntropySource::readByte() | "Entropy Error: Failed to read from serial port: <detail>" |
| SerialEntropySource::readEntropyPacket() | "Entropy Error: Timeout waiting for sync marker" |
| SerialEntropySource::reseed() | "Entropy Error: Reseeding not supported for hardware entropy source" |
| OpenSSLEntropySource::getEntropy() | "Entropy Error: RAND_bytes failed" |

Error reporting to consumers:

- Consumers of EntropyPool receive EntropyException on failure.

- Consumers of IEntropySource receive EntropyException on failure.

- The collector catches EntropyException and logs it before falling back.

9.13 Error Logging

All errors are logged through the backend's logger (Section 7.4). Log format is JSON lines to the log file and human-readable to console.

Log levels:

| Level | Usage |
| --- | --- |
| DEBUG | Sync loss/recovery, refill triggers, frame-level events |
| INFO | Source initialized, pool started/stopped, fallback recovery |
| WARN | Source unavailable, starvation, fallback activation |
| ERROR | Collection failure, shutdown errors, unrecoverable pool state |

Log entry structure:

json

{

"level": "WARN",

"timestamp": "2026-09-12T14:30:10Z",

"module": "SerialEntropySource",

"event": "read_timeout",

"message": "Read timed out waiting for bytes",

"details": {

"port": "COM3",

"timeout_ms": 5000

}

}

Exact field names are implementation-defined, but each entry must include: level, timestamp, module, event, and message.

9.14 Resilience Summary

| Failure | Detection | Response | Consumer Impact |
| --- | --- | --- | --- |
| Photodiode failure | Analyzer | Report diagnostic | None |
| Firmware reset | Serial timeout | Sync recovery; resume | Brief pause |
| Serial port unavailable | Open failure | OpenSSL fallback | None |
| Serial timeout | Read returns 0 | Retry; fallback after limit | Possible delay |
| Sync loss | Parser check | Sliding scan recovery | One misaligned byte |
| Pool starvation | getEntropy() blocks | Collector refills | Possible delay |
| Source exception | Caught by collector | OpenSSL fallback | None |
| Pool stopped | running flag | Throw EntropyException | Consumer handles |
| Config invalid | Startup validation | Fail to start | None (host not running) |
| Analyzer failure | Non-zero exit | Log error | None |
| Shutdown | Signal handler | Clean stop | None |

The subsystem is designed so that no single component failure causes a cascade. Each layer isolates its own errors and reports them upward in a controlled way.

# 10. Testing Strategy

10.1 Hardware Testing

Hardware testing verifies the analog front end before firmware is connected.

| Test | Method | Pass Criteria |
| --- | --- | --- |
| Vref voltage | Multimeter on Vref node | 1.65 V ± 5% |
| Op-amp supply | Multimeter on Pin 8 | 3.3 V ± 5% |
| Ground continuity | Multimeter between GND points | 0 Ω |
| Photodiode reverse bias | Multimeter on photodiode | Cathode at 3.3 V, anode lower |
| Op-amp output idle | Multimeter on Pin 1 and Pin 7 | Near Vref ± 100 mV |
| Op-amp output under light | Multimeter with flashlight | Output varies from idle |
| ADC reading idle | ESP32 analogRead() | Value near midrange (≈2048 for 12-bit) |
| ADC reading under light | ESP32 analogRead() with flashlight | Value changes noticeably |

Hardware test tools:

| Tool | Purpose |
| --- | --- |
| Multimeter | Voltage, continuity |
| Oscilloscope (optional) | Signal waveform, noise inspection |
| Adjustable light source | Controlled illumination |
| Blocking material | Darkness testing |

10.2 Firmware Testing

Firmware testing verifies that the ESP32 produces correctly framed bytes.

| Test | Method | Pass Criteria |
| --- | --- | --- |
| Serial output present | Serial monitor at 115200 | Bytes appear continuously |
| Sync marker cadence | Hex dump of serial output | 0xAA appears every 2 bytes |
| Frame structure | Hex dump | Every frame is [0xAA][payload] |
| Byte distribution | Collect 10,000 bytes, histogram | Roughly uniform |
| Von Neumann correctness | Feed biased sequence | Output bits are balanced |
| LED activity | Visual inspection | LEDs toggle at reasonable rate |
| LED throttling | Timing | LEDs update approximately every 100 ms |
| Reset recovery | Power cycle ESP32 | Stream resumes within seconds |

Firmware test tools:

| Tool | Purpose |
| --- | --- |
| Arduino Serial Monitor | Live stream inspection |
| xxd or hexdump | Hex-level frame inspection |
| Python script | Automated byte collection and histogram |

10.3 Serial Protocol Testing

Serial protocol testing verifies the host-side parser.

| Test | Method | Pass Criteria |
| --- | --- | --- |
| Normal parsing | Feed known good frames | Payload extracted correctly |
| Sync loss recovery | Feed misaligned frames | Parser recovers within a few bytes |
| False sync handling | Feed 0xAA in payload | Parser self-corrects within one frame |
| Timeout behavior | Stop feeding bytes | Read returns 0 within timeout |
| Port open failure | Open nonexistent port | initialize() returns false |
| Port close mid-stream | Close port during read | Read error, source marked unavailable |

Serial protocol test tools:

| Tool | Purpose |
| --- | --- |
| Unit tests | Feed synthetic byte streams to parser |
| socat (Linux) | Virtual serial port pair |
| com0com (Windows) | Virtual COM port pair |
| wscat equivalent | Not applicable; serial is not WebSocket |

10.4 Integration Testing

Integration testing connects adjacent layers.

| Test | Layers | Method | Pass Criteria |
| --- | --- | --- | --- |
| Firmware to parser | ESP32 → SerialEntropySource | Connect real ESP32, read bytes | Bytes flow continuously |
| Parser to pool | SerialEntropySource → EntropyPool | Start pool with serial source | Pool fills |
| Pool to consumer | EntropyPool → test consumer | Call getEntropy(n) | Returns n bytes |
| Fallback path | Collector with hardware down | Start with no ESP32 | Pool still serves from OpenSSL |
| Analyzer integration | Backend → Python subprocess | Invoke analyzer | JSON result returned |
| End-to-end | All layers | Full entropy path + analyzer | Analyzer passes |

10.5 Host-Side Entropy CLI Test Bench

To validate the backend entropy pool, cryptographic routines, and NIST randomness quality independently before full gateway integration, an interactive command-line test bench (EntropyCLI.exe, launched via scripts/runEntropy.bat) is provided. This standalone C++ utility interacts with the entropy subsystem to display real-time fill level and throughput speedometers, generate and format test random blocks (Hex, Base64, raw bitstreams, integer dice/coin rolls), test PBKDF2 and AES-256-GCM encryption/decryption round trips, toggle hardware/software entropy sources and whitening algorithms (Galois LFSR vs NIST SP 800-90B SHA-256), and execute the NIST SP 800-22 statistical test suite. In exhaustive test mode, the CLI generates a 10-bin p-value distribution histogram against the theoretical uniform distribution baseline to mathematically certify cryptographic randomness.

| Tool | Purpose |
| --- | --- |
| Real hardware | ESP32 + photodiodes |
| Virtual serial port | Simulate ESP32 stream |
| pytest | Python integration tests |
| C++ integration tests | End-to-end backend tests |

# 11. File and Directory Structure

11.1 Overview

This section defines the directory layout for the modules covered by this document: hardware firmware, backend entropy subsystem, analyzer, configuration, and build scripts. Frontend, crypto engine, session management, and gateway integration are excluded.

The layout is designed so that each module has a single clear location and can be developed, tested, and replaced independently. Paths are relative to the project root.

11.2 Relevant Directory Tree

PseudoQuantumEntropy/

│

├── hardware/

│   └── PQHardware.ino              # ESP32 firmware (Section 4)

│

├── backend/

│   ├── CMakeLists.txt              # Build configuration (Section 10)

│   ├── src/

│   │   ├── main.cpp                # Backend entry point

│   │   ├── EntropyCollector.cpp/h  # Source selection and coordination (Section 6.6)

│   │   ├── EntropyPool.cpp/h       # Circular buffer and collection thread (Section 6.7)

│   │   ├── Config.cpp/h            # Configuration loader (Section 7)

│   │   ├── Logger.cpp/h            # Logging (Section 7.4)

│   │   │

│   │   ├── entropy/

│   │   │   ├── IEntropySource.h    # Abstract interface (Section 6.3)

│   │   │   ├── SerialEntropySource.cpp/h    # Hardware source (Section 6.4)

│   │   │   └── OpenSSLEntropySource.cpp/h   # Fallback source (Section 6.5)

│   │   │

│   │   └── utilities/

│   │       └── Helpers.cpp/h       # Shared helpers (optional)

│   │

│   ├── include/

│   │   └── json.hpp                # nlohmann/json (header-only)

│   │

│   └── tests/

│       ├── test_entropy_pool.cpp

│       ├── test_serial_parser.cpp

│       ├── test_config.cpp

│       └── test_entropy_source.cpp

│

├── analyzer/

│   ├── requirements.txt            # Python dependencies (Section 8.11)

│   ├── entropy_analyzer.py         # Analyzer entry point (Section 8.3)

│   ├── nist_tests.py               # Statistical test implementations (Section 8.3)

│   └── test_results_schema.json    # JSON schema for analyzer output (Section 8.7)

│

├── config/

│   ├── backend_config.json         # Backend configuration (Section 7.2)

│   └── logging_config.json         # Logging configuration (Section 7.4)

│

├── scripts/

│   ├── build.sh                    # Build script (Linux/macOS)

│   └── run.bat                     # Run script (Windows)

│

├── logs/                           # Runtime log output (gitignored)

│   └── backend.log

│

└── data/                           # Runtime data (gitignored)

└── (device registry, out of scope)

11.3 Key File Descriptions

Hardware:

| File | Purpose |
| --- | --- |
| PQHardware.ino | Firmware: ADC sampling, LSB extraction, Von Neumann whitening, serial framing, LED status |

Backend entropy subsystem:

| File | Purpose |
| --- | --- |
| IEntropySource.h | Abstract interface for all entropy providers |
| SerialEntropySource.cpp/h | Reads ESP32 serial stream, parses frames, recovers sync, implements IEntropySource |
| OpenSSLEntropySource.cpp/h | Software fallback using RAND_bytes(), implements IEntropySource |
| EntropyCollector.cpp/h | Selects active source, detects availability, triggers fallback |
| EntropyPool.cpp/h | Thread-safe circular buffer, background collection, blocking and non-blocking reads |

Backend support:

| File | Purpose |
| --- | --- |
| main.cpp | Backend entry point; loads config, initializes logger, starts pool |
| Config.cpp/h | Parses backend_config.json and logging_config.json |
| Logger.cpp/h | Thread-safe logging with levels and file/console output |

Analyzer:

| File | Purpose |
| --- | --- |
| entropy_analyzer.py | Entry point; reads bytes, selects mode, invokes tests, emits JSON |
| nist_tests.py | Individual statistical test implementations |
| test_results_schema.json | JSON schema for analyzer output validation |
| requirements.txt | Python dependencies |

Configuration:

| File | Purpose |
| --- | --- |
| backend_config.json | Serial, entropy, analysis, and other backend settings |
| logging_config.json | Log file path, level, console output |

Build:

| File | Purpose |
| --- | --- |
| CMakeLists.txt | CMake build definition |
| build.sh | Convenience build script |
| run.bat | Windows run script |

11.4 Directory Design Principles

- One module per directory. Each logical module has a single home.

- Headers next to sources. No separate include/ for project code; only third-party headers live in include/.

- Tests in tests/. Unit and integration tests are separate from source.

- Configuration in config/. No config values embedded in source.

- Runtime data in logs/ and data/. Both are gitignored.

- Analyzer separate from backend. Python code lives in its own directory and is invoked as a subprocess.

- No vendored dependencies. OpenSSL, Boost, and header-only libraries are resolved by the build system

# 12. Future Considerations

12.1 Overview

This section outlines potential enhancements and improvements beyond the current specification. None of these are required for the initial implementation. They represent natural extensions of the architecture and should be considered only after the base system is validated.

Design principles for future work:

- Backward compatibility. New features should not break existing interfaces.

- Layer independence. Each enhancement should live in the layer where it belongs.

- No premature optimization. Add complexity only when justified by measurements.

- Honest claims. Any new claim about entropy quality must be validated by the analyzer.

12.2 Short-Term Enhancements

These are low-effort improvements that can be added without changing existing interfaces.

| Enhancement | Description | Priority |
| --- | --- | --- |
| Runtime hardware recovery | Automatically switch back to hardware source when serial port becomes available again after fallback | High |
| Configurable log format | Allow log output format (JSON, text, syslog) to be selected in logging_config.json | Medium |
| Pool statistics endpoint | Expose pool fill level, refill count, and source name for monitoring | Medium |
| Analyzer result caching | Cache the most recent analyzer result to avoid redundant runs | Low |
| Serial port auto-detection | Scan available serial ports and pick the ESP32 by VID/PID | Medium |
| Configurable inter-sample delay | Allow firmware to change the delay between ADC reads without recompilation | Low |
| Graceful shutdown logging | Log pool state (bytes collected, source used, uptime) on shutdown | Low |

12.3 Medium-Term Enhancements

These require more design but fit within the current architecture.

| Enhancement | Description | Priority |
| --- | --- | --- |
| Entropy health monitoring | Continuous lightweight statistical checks in the backend (not full NIST) to detect degradation in real time | High |
| Multi-source blending | Mix hardware and OpenSSL entropy using a cryptographic hash to improve robustness | Medium |
| Hardware watchdog integration | Detect ESP32 hangs via the serial protocol and trigger fallback faster | Medium |
| Configurable frame format | Support alternative frame formats (e.g., 0xAA 0x55 preamble) to reduce false sync risk | Low |
| Firmware self-test mode | A mode where the ESP32 outputs known patterns for testing parser behavior | Low |
| Remote analyzer trigger | Allow analyzer to be triggered via REST endpoint (out of scope here, but noted) | Medium |
| Cross-platform serial abstraction | Replace Boost.Asio with a lighter abstraction to reduce dependencies | Low |
| Deterministic test harness | A simulated entropy source for testing the pool without hardware or OpenSSL | High |
| Pool persistence across restarts | Save/restore pool contents to avoid startup starvation (with caution) | Low |
| Secure logging | Redact or hash sensitive fields in log entries | Low |

12.4 Long-Term Enhancements

These are significant additions that may require architectural changes.

| Enhancement | Description | Priority |
| --- | --- | --- |
| Additional entropy sources | Add support for other physical sources (ring oscillator, avalanche noise, etc.) | Medium |
| Hardware entropy certification | Pursue NIST SP 800-90B compliance for the hardware source | Low |
| Cryptographic whitening layer | Replace Von Neumann with a stronger extractor (e.g., SHA-256 based) for higher assurance | Medium |
| Formal verification of pool | Model-check the pool's thread safety and refill logic | Low |
| Hardware entropy measurement | Add a dedicated entropy measurement device for continuous monitoring | Low |
| Multi-device entropy aggregation | Combine entropy from multiple ESP32 units | Low |
| Tamper detection | Detect physical tampering with the analog front end | Low |
| Entropy budgeting | Track cumulative entropy and warn when the source is running low | Medium |
| FIPS compliance path | Align with FIPS 140-3 requirements for entropy sources | Low |

12.5 Security Considerations

The following security improvements are recommended as the system matures.

| Consideration | Description | Priority |
| --- | --- | --- |
| Raw byte confidentiality | Ensure raw entropy bytes are not logged or persisted | High |
| Pool zeroization | Overwrite pool memory on shutdown to prevent residual data leakage | High |
| Secure memory for keys | Use OS-level secure memory for sensitive buffers (out of scope for entropy pool but relevant downstream) | Medium |
| Log redaction | Redact serial port paths and device identifiers if logs are shared | Medium |
| Entropy source authentication | Verify the ESP32 identity via a challenge-response before trusting its stream | Low |
| Anti-rollback for firmware | Ensure the ESP32 runs known firmware | Low |
| Supply chain verification | Verify OpenSSL and Boost binaries | Medium |
| Threat model documentation | Document assumptions about the threat environment | High |

12.6 Error Detection Improvements

## Additional error detection beyond the current specification

| Improvement | Description | Priority |
| --- | --- | --- |
| Frame counter | Add an optional sequence number to frames to detect dropped bytes | Low |
| Payload checksum | Add an optional checksum byte to frames (increases frame size) | Low |
| Health score | Compute a running health score from read timeouts, sync loss, and pool fill rate | Medium |
| Anomaly detection | Detect statistically improbable sequences (e.g., repeated identical bytes) and log warnings | Medium |
| Source comparison | When both hardware and OpenSSL are available, compare outputs for anomalies | Low |
| ADC range monitoring | Firmware reports min/max ADC values periodically to detect saturation | Medium |
| Power supply monitoring | Monitor 3.3 V rail for droops (requires additional hardware) | Low |
| Self-test on startup | Run a brief internal consistency check on the pool at startup | Medium |

12.7 Performance Optimizations

| Optimization | Description | Expected Gain |
| --- | --- | --- |
| Lock-free buffer | Replace mutex-based circular buffer with a lock-free ring buffer | Reduced contention |
| Batched serial reads | Read multiple frames per serial call to reduce syscall overhead | Higher throughput |
| Pre-allocated buffers | Avoid per-call allocation in getEntropy() | Lower latency |
| Vectorized parsing | Parse multiple frames in a single pass | Lower CPU usage |
| Compiled analyzer | Rewrite analyzer in C++ for speed | Faster tests |
| Parallel test execution | Run analyzer tests in parallel | Faster analysis |
| Compiler optimizations | -O3 -march=native for production builds | Moderate speedup |

12.8 Operational Enhancements

| Enhancement | Description | Priority |
| --- | --- | --- |
| Metrics endpoint | Expose Prometheus-style metrics for pool state, source status, and error counts | Medium |
| Structured events | Emit structured events for monitoring systems (not just logs) | Medium |
| Alert thresholds | Trigger alerts when starvation or fallback events occur | Medium |
| Config versioning | Include a config schema version to support migrations | Low |
| Hot config reload | Reload backend_config.json without restart (limited to safe fields) | Low |
| Health check endpoint | Expose /health for container orchestration | Medium |
| Graceful degradation modes | Define explicit degraded modes (e.g., "hardware unavailable but functional") | Medium |

12.9 Research Directions

Longer-term research questions that could inform future versions.

| Direction | Description |
| --- | --- |
| Quantum contribution measurement | Quantify the fraction of entropy attributable to photon shot noise vs classical noise |
| Adversarial robustness | Evaluate the source's resistance to optical injection attacks |
| Entropy estimation theory | Apply NIST SP 800-90B estimators to the raw LSB stream |
| Hardware diversity | Test the design with different photodiodes, op-amps, and microcontrollers |
| Environmental sensitivity | Study entropy quality across temperature, humidity, and lighting conditions |
| Aging effects | Measure entropy quality degradation over long-term operation |
| Formal entropy accounting | Maintain a cumulative entropy budget with conservative estimates |
| Cross-platform reproducibility | Verify the same design produces similar entropy across different builds |

12.10 Out of Scope for Future Work

The following are explicitly excluded from the entropy subsystem, even in future versions:

- Cryptographic key generation (separate module)

- Session management (separate module)

- Web frontend (separate module)

- ML threat detection (separate module)

- Gateway integration (separate module)

The entropy subsystem's job is to produce random bytes. Anything built on top of that is a separate concern.

# 13. Limitations, References, and Glossary

13.1 Limitations and Claim Boundaries

Purpose. This subsection defines what the PseudoQuantum Entropy system can and cannot claim about the quality and origin of its output. It exists to prevent overstatement and to ensure that any downstream documentation, presentation, or publication uses accurate language.

The PseudoQuantum name

The system is called PseudoQuantum Entropy, not a Quantum Random Number Generator (QRNG). This naming is deliberate and must be preserved in all documentation.

The rationale:

- Photon shot noise is quantum in origin.

- The measured signal also contains classical noise from multiple sources.

- The system does not isolate or prove the quantum contribution.

- Calling it a QRNG would overstate what has been demonstrated.

What the system does claim

The following claims are supported by the design and can be made confidently:

| Claim | Basis |
| --- | --- |
| Uses optical fluctuations as the intended entropy source | Reverse-biased photodiodes respond to incident light |
| Conditions raw noise for digital extraction | LSB extraction and Von Neumann whitening |
| Produces a continuous stream of framed bytes | Firmware design and serial protocol |
| Buffers entropy in a thread-safe pool | EntropyPool design |
| Falls back to OpenSSL when hardware is unavailable | EntropyCollector behavior |
| Can be statistically validated | Python analyzer using NIST SP 800-22 |
| Provides cryptographically usable random bytes | Analyzer pass criteria (Section 8.8) |

What the system does not claim

The following claims are not supported and must not be made:

| Non-claim | Reason |
| --- | --- |
| "True quantum randomness" | Quantum contribution is not isolated or measured |
| "Provably quantum" | No measurement demonstrates quantum origin |
| "Certified QRNG" | No certification body has evaluated the system |
| "NIST SP 800-90B compliant" | Compliance has not been pursued or verified |
| "FIPS 140-3 validated" | No FIPS validation has been performed |
| "Information-theoretically secure" | Pool relies on computational assumptions and classical noise |
| "Immune to optical injection" | No adversarial robustness testing has been performed |
| "Uniformly random per byte" | Per-byte uniformity is validated statistically, not proven |

Noise sources in the measured signal

The measured entropy contains contributions from multiple physical and electrical sources. The relative proportions are not known without dedicated measurement.

| Source | Origin | Quantum? |
| --- | --- | --- |
| Photon shot noise | Discrete photon arrival at the photodiode | Yes |
| Thermal noise (Johnson–Nyquist) | Resistor thermal fluctuations | No |
| Op-amp input noise | LM358 internal noise | No |
| Flicker noise (1/f) | Semiconductor and resistor | No |
| Supply noise | 3.3 V rail fluctuations | No |
| ADC quantization noise | ESP32 ADC resolution | No |
| Ambient light fluctuation | Room lighting, air currents | No |
| Electromagnetic pickup | Breadboard wiring | No |

Photon shot noise is the only quantum-origin contributor. All others are classical.

Strongest defensible claim

The strongest claim the system can support, without further measurement, is:

The circuit uses optical fluctuations as the intended entropy source and conditions them for digital random-bit extraction. The output passes standard statistical tests for randomness. The system is not claimed to be a pure quantum random number generator.

This claim is accurate, defensible, and does not overstate what has been demonstrated.

Claim language guidelines

When describing the system in reports, papers, presentations, or documentation:

| Instead of | Use |
| --- | --- |
| "Quantum random number generator" | "PseudoQuantum entropy source" |
| "True quantum randomness" | "Optical fluctuations conditioned for entropy extraction" |
| "Provably random" | "Statistically validated" |
| "Quantum entropy" | "Optical entropy with quantum contribution" |
| "Certified entropy" | "Analyzer-passing entropy" |
| "Random bytes" | "Random bytes (statistically validated)" |

These substitutions preserve accuracy without diminishing the project's technical merit.

13.2 References

The following references are recommended reading and authoritative sources for the technologies and standards used by the entropy subsystem.

Statistical testing standards

| Reference | Description | Link |
| --- | --- | --- |
| NIST SP 800-22 Rev 1a | Statistical Test Suite for Random and Pseudorandom Number Generators | https://nvlpubs.nist.gov/nistpubs/Legacy/SP/nistspecialpublication800-22r1a.pdf |
| NIST SP 800-90B | Recommendation for the Entropy Sources Used for Random Bit Generation | https://nvlpubs.nist.gov/nistpubs/SpecialPublications/NIST.SP.800-90B.pdf |
| NIST SP 800-90A Rev 1 | Recommendation for Random Number Generation Using Deterministic Random Bit Generators | https://nvlpubs.nist.gov/nistpubs/SpecialPublications/NIST.SP.800-90Ar1.pdf |

Cryptography references

| Reference | Description | Link |
| --- | --- | --- |
| RFC 5116 | An Interface and Algorithms for Authenticated Encryption | https://tools.ietf.org/html/rfc5116 |
| FIPS 140-3 | Security Requirements for Cryptographic Modules | https://csrc.nist.gov/publications/detail/fips/140/3/final |
| OpenSSL Documentation | RAND_bytes() and cryptographic primitives | https://www.openssl.org/docs/ |

Hardware references

| Reference | Description | Link |
| --- | --- | --- |
| ESP32 Technical Reference Manual | ESP32 hardware including ADC | https://www.espressif.com/sites/default/files/documentation/esp32_technical_reference_manual_en.pdf |
| ESP32 ADC Documentation | ADC calibration and usage | https://docs.espressif.com/projects/esp-idf/en/latest/esp32/api-reference/peripherals/adc.html |
| LM358 Datasheet | Dual operational amplifier | https://www.ti.com/lit/ds/symlink/lm358.pdf |
| Arduino ESP32 Core | Arduino framework for ESP32 | https://github.com/espressif/arduino-esp32 |

Software references

| Reference | Description | Link |
| --- | --- | --- |
| Boost.Asio Documentation | Cross-platform serial I/O | https://www.boost.org/doc/libs/release/doc/html/boost_asio.html |
| nlohmann/json | JSON library for C++ | https://github.com/nlohmann/json |
| cpp-httplib | HTTP server library for C++ | https://github.com/yhirose/cpp-httplib |
| CMake Documentation | Build system | https://cmake.org/documentation/ |

Python and analyzer references

| Reference | Description | Link |
| --- | --- | --- |
| NumPy | Numerical operations | https://numpy.org/ |
| SciPy | Statistical functions | https://scipy.org/ |
| pytest | Python testing framework | https://docs.pytest.org/ |

Recommended reading

| Book / Paper | Author(s) | Year |
| --- | --- | --- |
| "Cryptography Engineering" | N. Ferguson, B. Schneier, T. Kohno | 2010 |
| "Introduction to Modern Cryptography" | J. Katz, Y. Lindell | 2020 |
| "The Design of Rijndael" | J. Daemen, V. Rijmen | 2002 |
| "Computer Security: Principles and Practice" | W. Stallings, L. Brown | 2018 |

13.3 Glossary

AAD. Additional Authenticated Data — metadata included in authentication but not encrypted. (Relevant to downstream crypto, not entropy pool.)

ADC. Analog-to-Digital Converter. The ESP32's ADC1 is used for both entropy channels.

ADC1. The ESP32 ADC peripheral used by GPIO34 and GPIO35. ADC1 is preferred over ADC2 because ADC2 is shared with Wi-Fi and cannot be used reliably while Wi-Fi is active.

Boost.Asio. A cross-platform C++ library for network and low-level I/O, used by SerialEntropySource for serial port access.

Circular buffer. A fixed-size buffer where writes wrap around to the beginning when the end is reached. Used by EntropyPool for in-memory entropy storage.

Collector. See EntropyCollector.

Condition variable. A synchronization primitive used with a mutex to block threads until a condition is satisfied. Used by EntropyPool for reader and collector coordination.

Entropy. Randomness used to generate cryptographic keys and other security-critical values.

Entropy budget. The estimated cumulative amount of entropy produced or consumed by a system.

EntropyCollector. The coordinator that selects the active entropy source, detects availability changes, and feeds the EntropyPool.

EntropyException. A subclass of std::runtime_error used for all entropy-related errors. Messages are prefixed with "Entropy Error: ".

Entropy pool. See EntropyPool.

EntropyPool. The thread-safe circular buffer that stores entropy and serves it to consumers.

ESP32. A microcontroller with Wi-Fi and Bluetooth, used as the ADC and serial interface for the hardware entropy source.

Fallback. The automatic switch from hardware entropy to OpenSSL when the hardware source is unavailable or starved.

Firmware. The ESP32 Arduino sketch (PQHardware.ino) that samples, whitens, and transmits entropy bytes.

Frame. A 2-byte serial packet: [0xAA][DATA_BYTE].

GCM. Galois/Counter Mode — a block cipher mode providing encryption and authentication. (Relevant to downstream crypto, not entropy pool.)

GPIO. General-Purpose Input/Output. ESP32 pins 34 and 35 are analog inputs on ADC1.

IEntropySource. The abstract interface implemented by all entropy providers.

IV. Initialization Vector. See Nonce.

Jitter. Timing variation in a signal. Not used as an entropy source in this design.

LM358. A dual operational amplifier used for analog amplification of photodiode signals.

LSB. Least Significant Bit. The lowest-order bit of an ADC reading, used as the raw entropy sample.

Mutex. A mutual exclusion primitive used to protect shared state. Used throughout EntropyPool.

NIST SP 800-22. A statistical test suite for random and pseudorandom number generators. Used by the analyzer.

NIST SP 800-90B. A standard for entropy sources used for random bit generation.

Nonce. Number used once. (Relevant to downstream crypto, not entropy pool.)

OpenSSL. A cryptographic library providing RAND_bytes() and other primitives. Used as the software fallback source.

OpenSSLEntropySource. The software fallback implementation of IEntropySource using RAND_bytes().

Operational amplifier. A high-gain electronic voltage amplifier. The LM358 is used here as a current-to-voltage converter.

Photodiode. A semiconductor device that converts light into electrical current. Two are used in this design, one per channel.

Photon shot noise. Quantum-origin noise arising from the discrete nature of photon arrival. A contributor to the measured entropy, but not the only one.

PseudoQuantum. The term used to describe the entropy source in this project. Indicates that the source has quantum contributions but is not claimed to be purely quantum.

Refill threshold. The buffer fill level below which the collector thread initiates a refill. Configurable via pool_refill_threshold.

Reverse bias. A photodiode operating mode in which the cathode is at higher potential than the anode. Increases depletion width and sensitivity.

SerialEntropySource. The hardware entropy source implementation of IEntropySource, reading the ESP32 serial stream.

Shot noise. See Photon shot noise.

Sync marker. The byte 0xAA used as a frame delimiter in the serial protocol.

Thermal noise. Johnson–Nyquist noise arising from thermal fluctuations in resistors. A classical contributor to the measured entropy.

TRNG. True Random Number Generator. The system is not claimed to be a pure TRNG.

Vref. The 1.65 V midpoint reference used by the analog front end, generated by a 150 kΩ / 150 kΩ divider.

Von Neumann extractor. A debiasing technique that discards equal bit pairs and outputs only when the pair differs. Used in firmware.

Whitening. Any process that removes bias or correlation from a bit stream. The firmware uses Von Neumann whitening.